3GPP TSG-RAN WG4 Meeting#119                              R4-2607243

Dalian, CN, May 18 - 22, 2026

Agenda item:			8.1.1

Source:	Moderator (ZTE)

Title:	Topic summary for [119][362] 6G_Sensing_coex_test_req

Document for:	Information

- 1 Introduction

Based on the latest approved RAN working group level SID on 6G Radio [RP-253876], RAN4 study on sensing including RF, coexistence, testability, with the detailed objective provided as:

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

In the last RAN4#118bis meeting, based on the initial discussions on 6G sensing, we reached the following consensus as captured in WF R4-2605009:

Issue 1-1: How to capture 6G sensing study in RAN4 Agreement: - To capture the 6G sensing study including coexistence study, feasibility study etc in 6G RAN4 TR38.760-4 (Study on 6G Radio RAN4 aspects) Issue 1-2: Regulatory requirements Agreement: - To capture the regulatory requirements in 6G TR for the information only; - Keep monitoring regulatory requirements from different regions and provide the corresponding updates in RAN4; Issue 2-2: Use case and Issue 2-3: Sensing mode Agreement for the scenarios (sensing modes and sensing target) for 6G sensing coexistence study from RAN4 work management perspective: - UAV (passive object), TRP monostatic - FFS on Multiple TRP monostatic. - Vehicle (passive object), TRP-UE bistatic - [TBD] (passive object), UE-TRP bistatic - UAV (passive object), TRP-TRP bistatic RAN4 doesn’t preclude any 6G scenarios as agreed by RAN-P and RAN1; Agreement for the feasibility study for 6G sensing: - UAV (passive object), TRP monostatic - FFS on Multiple TRP monostatic. - Vehicle (passive object), TRP-UE bistatic - [TBD] (passive object), UE-TRP bistatic - UAV (passive object), TRP-TRP bistatic - In addition to above cases, other sensing modes/scenarios as agreed by RANP and RAN1 could be also studied. Issue 3-1: General assumption for the feasibility study Agreement for the prioritized bands to be investigated for 6G sensing - FR1 TDD bands +FR2-1 TDD bands; - Encourage inputs from the operators for the bands; Agreement: - RAN4 to use the same 6GR BS and UE assumptions for 6G sensing study as starting point; Issue 3-2: Feasibility study for TRP/UE mono-static Agreement: - For the feasibility study of mono-static (TRP, UE), propose to consider the following factors: - Whether to consider the Tx/Rx separate panels for TRP; - Whether to consider the Tx/Rx separate antennas for UE; - FFS on the switching time for Tx/Rx switching; - FFS on Self interference - Quality of received reflected signal - Receiver linearity and blocking etc - Others are not precluded. - The operation mode/sensing waveform depends on the design from other WGs. Issue 3-3: Feasibility study for TRP-UE and/or UE-TRP bi-static Agreement: - For the feasibility study of TRP-UE and/or UE-TRP bi-static, propose to consider the following factors: - FFS for Sensing node’s receiver dynamic range - Quality of received reflected signal - Synchronization between TRP and UE - Others are not precluded. Issue 6-2: The BS type for 6G conformance testing Agreement: - For TRP mono-static and bi-static sensing mode, - BS type 1-O and BS type 2-O need to be studied. - FFS on BS type 1-H

In the running summary [R4-2606889], we also captured the agreement reached for sensing in RAN/RAN1/RAN2 to provide the background information to facilitate RAN4’s sensing discussion. In this thread, we summarized the contribution submitted under the agenda 8.10 for the discussion.

- 2 Topic: 6G sensing

- 2.1. Summary for Coexistence study and simulation assumptions

Issue 1-1: General for coexistence case

- Proposal 1: Considering it is early stage for 6G study and the system design is premature, NR can be used as the communication system in the coexistence study with sensing. [Huawei]

- Proposal 2: For coexistence study for 6G sensing, propose to consider the coexistence cases as listed in Table 2.1-1. [ZTE]

| Coexistence case | Aggressor | Victim | Remark |
| --- | --- | --- | --- |
| 1 | 6G sensing | 6GR communication | if the transmitter of sensing signal regardless of TRP or UE can comply with IMT BS/UE requirements, then no coexistence study is expected. |
| 2 | 6GR communication | 6G sensing | Coexistence study is needed since the impacts of 6GR adjacent channel leakage on sensing results are not clear, this needs the corresponding coexistence study. |
| 3 | 6G sensing | 5G NR communication | if the transmitter of sensing signal regardless of TRP or UE can comply with IMT BS/UE requirements, then no coexistence study is expected. |
| 4 | 5G NR communication | 6G sensing | Coexistence study is needed since the impacts of 5G adjacent channel leakage on sensing results are not clear, this needs the corresponding coexistence study. |
| 5 | 6G sensing | 6G Sensing | Similar as coexistence 2. |

- Proposal 3: RAN4 to investigate coexistence of an ISAC network with 5G/6G networks according to below Table with decreasing priority from items 1 to 5, thereby prioritizing the study of coexistence between 6G ISAC network as victim network and existing deployed networks as aggressor networks, such as legacy NR, and thereafter prioritizing 6G ISAC network as aggressor network and existing deployed networks as victim networks to ensure protection of current and envisaged future deployments. [Nokia]

| - Coexistence scenario | Aggressor signal | Victim signal |
| --- | --- | --- |
| 1 | Legacy NR | 6G ISAC |
| 2 | 6G | 6G ISAC |
| 3 | 6G ISAC | 5G |
| 4 | 6G ISAC | 6G |
| 5 | 6G ISAC | 6G ISAC |

Scenarios for ISAC coexistence study (decreasing priority).

- Proposal 4: RAN4 to down-select co-existence scenarios for 6G sensing co-existence evaluation when sufficient implementation details have been made by RAN1&RAN4. [Samsung]

- Observation 1: There would be at least three typical types of co-existence cases in ISAC co-existence study, which are 1) 5G NR legacy communication system – ISAC system co-existence, 2) 6G legacy communication system – ISAC system co-existence, and 3) ISAC system – ISAC system co-existence. [Samsung]

- Proposal 5: RAN4 to consider the co-existence aggressor-victim cases listed in Table 2.1.2-1 as starting point in 6G sensing co-existence study. [Samsung]

Table 2.1.2-1: Adjacent channel aggressor-victim cases

| Case | Aggressor | Aggressor baseline | Victim |
| --- | --- | --- | --- |
| 1 | ISAC TRP Tx | NR TDD DL | NR TDD DL |
| 2 | ISAC TRP Tx | NR TDD UL | NR TDD UL |
| 3 | ISAC UE Tx | NR TDD DL | NR TDD DL |
| 4 | ISAC UE Tx | NR TDD UL | NR TDD UL |
| 5 | ISAC TRP Tx | 6G TDD DL | 6G TDD DL |
| 6 | ISAC TRP Tx | 6G TDD UL | 6G TDD UL |
| 7 | ISAC UE Tx | 6G TDD DL | 6G TDD DL |
| 8 | ISAC UE Tx | 6G TDD UL | 6G TDD UL |
| 9 | NR TDD DL | No Aggressor Network | ISAC TRP Rx |
| 10 | NR TDD UL | No Aggressor Network | ISAC TRP Rx |
| 11 | NR TDD DL | No Aggressor Network | ISAC UE Rx |
| 12 | NR TDD UL | No Aggressor Network | ISAC UE Rx |
| 13 | 6G TDD DL | No Aggressor Network | ISAC TRP Rx |
| 14 | 6G TDD UL | No Aggressor Network | ISAC TRP Rx |
| 15 | 6G TDD DL | No Aggressor Network | ISAC UE Rx |
| 16 | 6G TDD UL | No Aggressor Network | ISAC UE Rx |
| 17 | ISAC TRP Tx | No Aggressor Network | ISAC TRP Rx |
| 18 | ISAC TRP Tx | No Aggressor Network | ISAC UE Rx |
| 19 | ISAC UE Tx | No Aggressor Network | ISAC TRP Rx |
| 20 | ISAC UE Tx | No Aggressor Network | ISAC UE Rx |

- Observation 2: RAN4 has to further down-select co-existence aggressor-victim cases for 6G sensing co-existence evaluation by accounting the importance and urgency of each case. [Samsung]

- Proposal 6: RAN4 to deprioritize the ISAC–ISAC co-existence cases (case 17-20) in current 6G sensing co-existence study. [Samsung]

- Proposal 7: RAN4 to treat the UE related sensing co-existence cases (case 3, 4, 7, 8, 11, 12, 15, 16) with secondary priority according to current work load. [Samsung]

- Proposal 8: RAN4 should prioritize the cases of the ISAC–NR co-existence (case 1, 2) and the ISAC–6G co-existence (case 5, 6), which can be summarized as Table 2.1.2-2 [Samsung]

Table 2.1.2-2: Prioritized adjacent channel aggressor-victim cases

| Case | Aggressor | Aggressor baseline | Victim |
| --- | --- | --- | --- |
| 1 | ISAC TRP Tx | NR TDD DL | NR TDD DL |
| 2 | ISAC TRP Tx | NR TDD UL | NR TDD UL |
| 5 | ISAC TRP Tx | 6G TDD DL | 6G TDD DL |
| 6 | ISAC TRP Tx | 6G TDD UL | 6G TDD UL |

- Proposal 9: RAN4 to prioritize coexistence scenarios on cases where ISAC network is the victim given the novelty on how to model such cases within RAN4.  [Qualcomm]

- Proposal 10: To focus on the legacy system interfering ISAC system in adjacent channel co-ex if time is limited. [Vivo]

- Proposal 11: To start with TRP-UE and UE-TRP bistatic scenarios. [Vivo]

- Proposal 12: RAN4 to discuss if potential cross interference scenarios within a single operator/ channel will be investigated by RAN4 or not.  [Qualcomm]

- Recommended WF:

  - Propose to consider the following coexistence cases as starting point

| - Coexistence scenario | Aggressor signal | Victim signal | NOTE |
| --- | --- | --- | --- |
| 1 | Legacy NR | 6G ISAC | Novel and prioritized |
| 2 | 6GR | 6G ISAC | Case 2 is expected to be similar as Case 1 |
| 3 | 6G ISAC | Legacy NR |  |
| 4 | 6G ISAC | 6GR | Case 4 is expected to be similar as Case 3 |
| 5 | 6G ISAC | 6G ISAC |  |

  - Further discuss the priority for the above coexistence scenarios

    - Scenarios for ISAC coexistence study (decreasing priority). [Nokia]

    - Others

  - Further discuss whether to consider the co-channel UE2UE CLI problems in RAN4 [Qualcomm];

Issue 1-2: Operation frequency for 6G sensing

- Proposal 1: RAN4 to consider 7 GHz and 30 GHz as exemplary bands for FR1 and FR2-1 TDD bands in the ISAC coexistence study.   [Qualcomm]

- Proposal 2: RAN4 to reuse simulation assumptions in TR38.922 for NR network in 6G ISAC-NR coexistence evaluation and adopt 4GHz as starting point. [Samsung]

- Proposal 3: RAN4 to reuse simulation assumptions in 6G TN ~7GHz co-existence study in 6G ISAC-6G coexistence evaluation, which is summarized in Table 2.2.2. [Samsung]

- Proposal 4: The simulation assumptions in 6G TN ~7GHz co-existence can be adopted as a starting point for sensing network parameters. If specific parameters are unavailable, those in TR 38.922 could be used instead. [Samsung]

- Proposal 5: For the use case and sensing mode, propose to consider the above deployment scenario for coexistence study and further discuss the down-selection on the deployment e.g. pick up one of two deployment scenarios.  [ZTE]

| Use case | Deployment | Carrier frequency |
| --- | --- | --- |
| UAV detection and tracking including both mono-static and bi-static | UMa-AV, SMa | 4 GHz, ~7 GHz, 30 GHz (for UMa-AV only) |
| Vehicle detection and tracking | Urban Grid, Highway | 4 GHz, ~7 GHz, 30 GHz (for Urban Grid only) |

- Proposal 6: Use for the 4 GHz UAV and BS-to-BS bistatic and BS monostatic the parameters from the Table 2.2.1-1a to Table 2.2.1-1e and Table 2.2.2-1b [Ericsson]

- Proposal 7: RAN4 to investigate coexistence for following frequency bands according to below Table with decreasing priority from items 1 to 3. [Nokia]

| - Coexistence scenario | Frequency band | Other parameters |
| --- | --- | --- |
| 1 | 4 GHz | macro cell |
| 2 | 7 GHz (up to 8.4 GHz) | macro cell |
| 3 | 26 GHz | macro cell |

Table 4: Use case scenarios and sensing modes for ISAC coexistence study (decreasing priority).

- Recommended WF:

  - Propose to consider the following operation frequency for 6G sensing coexistence study from the work management perspective

    - 4GHz

    - ~7GHz

    - 30GHz

Issue 1-3: Deployment scenarios for 6G sensing

- Proposal 1: For the use case and sensing mode, propose to consider the above deployment scenario for coexistence study and further discuss the down-selection on the deployment e.g. pick up one of two deployment scenarios.  [ZTE]

| Use case | Deployment | Carrier frequency |
| --- | --- | --- |
| UAV detection and tracking including both mono-static and bi-static | UMa-AV, SMa | 4 GHz, ~7 GHz, 30 GHz (for UMa-AV only) |
| Vehicle detection and tracking | Urban Grid, Highway | 4 GHz, ~7 GHz, 30 GHz (for Urban Grid only) |

- Proposal 2: RAN4 to investigate coexistence for following frequency bands according to below Table with decreasing priority from items 1 to 3. [Nokia]

| - Coexistence scenario | Frequency band | Other parameters |
| --- | --- | --- |
| 1 | 4 GHz | macro cell |
| 2 | 7 GHz (up to 8.4 GHz) | macro cell |
| 3 | 26 GHz | macro cell |

Table 4: Use case scenarios and sensing modes for ISAC coexistence study (decreasing priority).

- Observation 1: For IMT-2030, Urban Macro and Indoor Factory deployment scenarios were considered to define key performance metrics for sensing networks. [Nokia]

- Observation 2: In the RAN1 ISAC study, limiting the scope to the UAV use case, also the Rural Macro deployment scenario was optionally considered. [Nokia]

- Proposal 3: RAN4 to investigate coexistence of an ISAC network with 5G/6G networks for outdoor deployment scenarios. Other deployment scenarios are not precluded.  [Nokia]

- Proposal 4: Further discussion is needed on how rural area legacy TDD and ISAC scenario may look like considering that legacy TDD spectrum around 3.5 GHz is not very common in rural areas. [Ericsson]

- Observation 2: Rural deployment scenario could be valid in 6G sensing co-existence study. [Samsung]

- Proposal 5: RAN4 to consider the coexistence evaluation scenarios listed in Table 2.1.1 as starting point in 6G sensing co-existence study. [Samsung]

Table 2.1.1: Adjacent channel co-existence scenarios

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro |
| 2 | FR1 | Urban Hotspot | Urban Hotspot |
| 3 | FR1 | Indoor | Indoor |
| 4 | FR1 | Urban Macro | Micro |
| 5 | FR1 | Micro | Micro |
| 6 | FR1 | Rural | Rural |
| 7 | FR2-1 | Urban Macro | Urban Macro |
| 8 | FR2-1 | Urban Hotspot | Urban Hotspot |
| 9 | FR2-1 | Urban Dense | Urban Dense |
| 10 | FR2-1 | Indoor | Indoor |

- Proposal 6: RAN4 to agree on UMa-AV for UAV use case and further discuss which scenario (highway and/or urban grid) for vehicular use case for TP-UE bistatic coexistence. [Qualcomm]

- Proposal 7: To adopt Urban Macro and Indoor scenarios first in RAN4 co-ex studies, and to FFS how to consider Urban Grid in RAN4 co-ex study. [Vivo]

- Proposal 8: Use for the 4 GHz UAV and BS-to-BS bistatic and BS monostatic the parameters from the Table 2.2.1-1a to Table 2.2.1-1e and Table 2.2.2-1b [Ericsson]

Table 2.3.1-1a Network layout and system parameters

| Parameters | Value |
| --- | --- |
| Frequency and bandwidth | 4 GHz (covering the 3-4 GHz range) with 100 MHz bandwidth |
| Network layout | UMa/SMa: hexagonal grid, 19 macro sites, 3 sectors per site with wrap around (or victim centre sector) |
| Inter-site distance | 0.45 km (urban) and 0.9 km (suburban) Table 4.2.1.1-1 in TR 38.921 |
| Multi operator’s layout | UMa/SMa: 0%, 10%, 50% and 100% grid shift |
| TDD DL/UL ratio | 80/20% |
| Synchronised/unsynchronised TDD operation in multi-operator context | Compatible frame structure to align uplink/downlink switching points. ISAC can use DL time slot for transmitting and receiving sensing signals |
| Cell load | UMa/SMa interferer: 10%, 50% and full buffer |

- Table 2.3.1-1d UAV parameters

| Parameters | Value |
| --- | --- |
| UAV distribution | Uniform distribution with heights 25 to 300 meters (TR 38.901 / TR 36.777) |
| Minimum BS - UAV distance | 10 meters 3D (TR 38.901 / TR 36.777) |

- Recommended WF:

  - For TRP mono-static, discuss the following deployment scenario:

    - UMa-AV(LTE term used in TR 36.777) or UMa

    - SMa

      - Applicable for FR1

      - FFS for FR2-1

    - FFS for RMa-AV or RMa (LTE term used in TR 36.777)

      - Applicable for FR1

      - FFS for FR2-1

  - For TRP bi-static, discuss the following deployment scenario:

    - UMa-AV(LTE term used in TR 36.777) or UMa

    - SMa

      - Applicable for FR1

      - FFS for FR2-1

    - FFS for RMa-AV or RMa (LTE term used in TR 36.777)

      - Applicable for FR1

      - FFS for FR2-1

  - For TRP-UE bi-static, discuss the following deployment scenario:

    - Highway (as defined in the A.1.2 of TR 36.885)

    - Urban grid  (as defined in the A.1.2 of TR 36.885)

    - UMa

    - SMa

  - For UE-TRP bi-static, discuss the following deployment scenario:

    - Highway (as defined in the A.1.2 of TR 36.885)

    - Urban grid  (as defined in the A.1.2 of TR 36.885)

    - UMa

    - SMa

Issue 1-4: Simulation assumptions for TRP mono-static

- Proposal 1: For coexistence study, sensing mode of TRP monostatic is considered and starts with the following coexistence cases. [Huawei]

| Sensing mode and sensing target | index | Aggressor | victim |
| --- | --- | --- | --- |
| UAV, TRP monostatic | a | Target reflection | NR DL |
|  | b | Sensing signal | NR DL |
|  | c | NR DL | Target reflection |

- Proposal 2: Before simulation evaluation, RAN4 need to further analyse case (a) with target reflection as the aggressor in scenario#1 & #2 on the reflection power level. [Huawei]

|  | Aggressor system | Victim system |
| --- | --- | --- |
| Case 1 | 6G sensing Tx | 6G sensing Rx |
| Case 2 | 6G sensing Tx | TDD DL |
| Case 3 | TDD DL | 6G sensing Rx |
| Case 4 | 6G sensing Tx | TDD UL |
| Case 5 | TDD UL | 6G sensing Rx |

- Proposal 3: For UAV (passive object) + TRP monostatic including Multiple TRPs monostatic, the following adjacent channel coexistence cases are summarized for further discussion whether coexistence study is required in 6G sensing. [CATT]

- Proposal 4: It is proposed to further study the SI modelling for TRP monostatic, including assumptions on antenna isolation, analog/digital cancellation capabilities, and residual SI floor, which are prerequisites for link budget and sensitivity loss analysis. [Vivo]

- Proposal 5: RAN4 to discuss whether both TDM-based (i.e., Half-duplex) or Full-duplex based monostatic sensing or only one mode will be considered during the RAN4 ISAC coexistence study.   [Qualcomm]

- Proposal 6: RAN4 to further study the sensing transmitter modelling for the burst signal transmission scheme.[Samsung]

- Proposal 7: For the TRP mono-static, the coexistence studies between 6G sensing and the legacy system need to be conducted to figure out the appropriate RF requirements.  [ZTE]

Table 2.2.1-2: Adjacent channel co-existence cases for TRP mono-static deployment

| Case | Aggressor | Aggressor baseline | Victim | Remarks |
| --- | --- | --- | --- | --- |
| 1 | ISAC | TDD DL | TDD DL | Observation 1: For 6G sensing TRP interfering 6GR DL, if the sensing TRP can comply with 6GR BS requirements, then the coexistence performance between 6G sensing and 6G communication or 5G communication can be guaranteed. |
| 2 | TDD DL | No aggressor network | ISAC | Observation 2: For 6GR DL interfering 6GR sensing, the BS2BS CLI including reflected path and non-reflected path should be considered for the TRP monostatic with two implementation schemes. |
| 3 | ISAC | No aggressor network | ISAC | Observation 3: For 6GR sensing interfering 6GR sensing, the BS2BS CLI should be considered for the TRP monostatic with the implementation option 1 at least; Observation 4: For 6GR sensing interfering 6GR sensing, the BS2BS CLI is not considered for the TRP monostatic with the implementation option 2 if TDM sensing pattern are aligned from the different carriers; |
| NOTE: assume the ISAC to be deployed in the DL symbols to ensure the coverage as 1st priority |  |  |  |  |

- Proposal 8: for the simulation assumption for UAV sensing evaluation in RAN4, propose to use the above simulation assumptions for 6G sensing coexistence study.  [ZTE]

- Proposal 9: for the simulation assumption for TN BS, propose to follow the simulation assumption as captured in  TR 38.922 for FR1 TDD network and TR 38.858 for FR2 TDD network. [ZTE]

- Proposal 10: for the RCS of sensing target, propose to consider the RCS model as captured in clause 7.9.2.1 of TR 38.901 and simplify the RCS in the large scale pathloss model if necessary. [ZTE]

- Proposal 11: For the UAV use case of 6G ISAC study at least, propose to consider the distance, angle estimation and velocity estimation as the basic metric for the study in RAN4. [ZTE]

- Proposal 12: For the UAV use case of 6G ISAC study at least, propose to consider the distance accuracy impacts, angle estimation accuracy impacts and velocity estimation accuracy impacts to quantify the performance impacts from both co-channel and adjacent channel. [ZTE]

- Proposal 13: for the 6G ISAC coexistence study in RAN4, for 6GR performance metric, propose to use the legacy throughput loss as basic metric.  [ZTE]

- Proposal 14: The self-interference/intra-site and inter-site co-channel interference (multiple BSs) is discussed in feasibility studies and needs to be also considered in the coexistence simulation to understand the combined impact. [Ericsson]

- Proposal 15: UMa network layout with hexagonal grid, 19 macro sites and 3 sectors per site for victim and interferer (simplified with sectors boresights from both operators in same direction). Overlapping cell grids with cell shift: 0%, 10%, 50% and 100%. For the colocation there is ongoing work in Rel-20 BS RF evolution WI to define relevant assumptions. Cell loads for the interferer with: 10%, 50% and full buffer. Full buffer is the reference case for legacy TDD BS operation and conformance tests, for the new feature with ISAC operation at lower load situations in the network and the sensing performance requirements needs still be defined and understood. [Ericsson]

- Proposal 16: Single pulse as baseline to compute SINR [Ericsson]

- Proposal 17: The simulation results should compare the SINR CDFs with and without interference [Ericsson]

- Proposal 18: Sensitivity analysis for detection and tracking accuracy considering number of pulses and pulse integration for different interference level low/high situations [Ericsson]

- Proposal 19: Detection probability versus SINR is needed to define acceptable SINR threshold value [Ericsson]

- Proposal 20 Use for the 4 GHz UAV and BS-to-BS bistatic and BS monostatic the parameters from the Table 2.2.1-1a to Table 2.2.1-1e and Table 2.2.2-1b [Ericsson]

- Table 2.3.1-1a Network layout and system parameters

| Parameters | Value |
| --- | --- |
| Frequency and bandwidth | 4 GHz (covering the 3-4 GHz range) with 100 MHz bandwidth |
| Network layout | UMa/SMa: hexagonal grid, 19 macro sites, 3 sectors per site with wrap around (or victim centre sector) |
| Inter-site distance | 0.45 km (urban) and 0.9 km (suburban) Table 4.2.1.1-1 in TR 38.921 |
| Multi operator’s layout | UMa/SMa: 0%, 10%, 50% and 100% grid shift |
| TDD DL/UL ratio | 80/20% |
| Synchronised/unsynchronised TDD operation in multi-operator context | Compatible frame structure to align uplink/downlink switching points. ISAC can use DL time slot for transmitting and receiving sensing signals |
| Cell load | UMa/SMa interferer: 10%, 50% and full buffer |

Table 2.3.1-1c UE parameters (mainly from Section 4.3 in TR 38.922)

| UE Parameters | Value |
| --- | --- |
| Outdoor/indoor | Outdoor and indoor with 20/80% |
| Antenna | 1Tx/2Rx |
| Antenna gain | Isotropic: 0 dBi |
| Max Tx power | 23 dBm |
| Power control | Section 9.1 TR 36.942 |
| SNR target | 15 dB |
| Noise figure | 9 dB |
| Antenna height | 1.5 m for outdoor |
| Distribution (horizontal) | Uniform |
| Minimum BS - UE distance (2D) | 35 m For Uncoordinated deployment, minimum distance applies also to UEs and BS across different deployments. |

Table 2.3.1-1d UAV parameters

| Parameters | Value |
| --- | --- |
| UAV distribution | Uniform distribution with heights 25 to 300 meters (TR 38.901 / TR 36.777) |
| Minimum BS - UAV distance | 10 meters 3D (TR 38.901 / TR 36.777) |

Table 2.3.1-1e Channel models

| Parameters | Value |
| --- | --- |
| BS-UE channel model | UMa (Table 7.4.1-1 in TR 38.901) |
| BS-BS channel model | UMa TR 38.901. LoS model is the default defined for UMa For BS-to-BS interference Line of Sight (LoS) probability with 100% or FSPL |
| BS-UAV channel model | Rel-19 extensions for ISAC in TR 38.901 - Focusing on large scale parameters - RCS of the sensing target, the model captured in clause 7.9.2.1 |

Table 2.3.2-1b BS parameters (mainly from Section 4.2 and 4.4 in TR 38.922)

| Parameters | Value |
| --- | --- |
| BS power TRP | 37, 52 dBm (for Tx, Rx antenna isolation of 65, 80 dB respectively) (TR 38.765) |
| BS antenna height | UMa/SMa: 25 m |
| Antenna mechanical tilt | UMa/SMa: 6 degrees downtilt |
| AAS antenna elements | Table 4.4.1.2-1 in Reference TR 38.922 (for starting point) |
| ACLR/ACS | 45/46 dB |

- Proposal 21: RAN4 to consider the ISAC channel model in TR 38.901, clause 7.9 for 6G sensing coexistence study. [Nokia]

- Proposal 22: RAN4 to focus on large scale calibration in TR 38.901, clause 7.9.6.1, for 6G sensing coexistence study, and consider BS to BS link to be in LoS condition for BSs in vicinity. [Nokia]

- Proposal 23: RAN4 to take simulation assumptions in Table 7.9.6.1-1 of TR 38.901 as baseline for the UAV use case for TRP monostatic and TRP-TRP bistatic sensing modes in the 6G sensing coexistence study. [Nokia]

- Proposal 24: For the agreed FR1 feasibility study, the channel bandwidth should be configured to be greater than 100 MHz to meet the ranging resolution requirements. [BUPT, China Telecom]

- Proposal 25: To satisfy the 1m/s velocity resolution requirement for UAV targets in FR1 bands, the coherent accumulation time for velocity measurement simulations should be carefully configured. As shown by the comparative baseline analysis (which requires >77ms at lower frequencies), sufficient coherent accumulation time should be assumed to ensure reliable velocity estimation across different FR1 scenarios. [BUPT, China Telecom]

- Proposal 26: When designing ISAC frame structures and reference signal patterns, an equidistant distributed symbol arrangement in the time domain should be assumed, as it is more beneficial for velocity measurement than a centralized symbol arrangement. [BUPT, China Telecom]

- Proposal 27: RAN4 to consider the relevant large scale ISAC channel model given in Section 7.9 in TR 38.901 (Table 7.9.6.1-1 could be considered as a starting point) in the RAN4 ISAC coexistence study. [Qualcomm]

- Proposal 28: RAN4 to discuss the pros and cons of approximation framework based on theoretical closed form expressions that could be used to quantify impact of adjacent channel interference on the sensing performance. [Qualcomm]

- Proposal 29: RAN4 to investigate coexistence for following use case scenarios and sensing modes according to below Table with decreasing priority from items 1 to 4. [Nokia]

| - Coexistence scenario | Use Case Scenario | Sensing mode |
| --- | --- | --- |
| 1 | UAV | TRP monostatic |
| 2 | UAV | TRP-TRP bistatic |
| 3 | Vehicular | UE-TRP bistatic |
| 4 | Vehicular | TRP-UE bistatic |

Table 3: Use case scenarios and sensing modes for ISAC coexistence study (decreasing priority).

- Proposal 30: RAN4 can consider simulate Level D for the coexistence study. [Xiaomi]

- Observation 3: Conditional Detection Probability can be calculated based on SINR. [Xiaomi]

- Observation 4: False alarm probability can be calculated in the Level D simulation. [Xiaomi]

- Proposal 31 RAN4 can discuss to introduce small-scale parameters to simulate the sensing estimation error during coexistence study as below: [Xiaomi]

- Proposal 32: RAN4 can consider the following Level D based Sensing Coexistence Simulation Methodology. [Xiaomi]

- Proposal 33: RAN4 needs to discuss whether all the metrics shall be used as the evaluation criteria for coexistence. [Xiaomi]

- Proposal 34: RAN4 may select the most representative metric as the protection criterion. [Xiaomi]

- Proposal 35: RAN4 can discuss the protection criterion from the perspective of filter missed detection probability and sensing accuracy [Xiaomi]

- Proposal 36: Consider parameters and network layout in TR38.765 as the starting point of the discussion. [Xiaomi]

- Proposal 37: Consider channel model and target model in TR38.901 as the assumption for ISAC coexistence study. [Xiaomi]

- Proposal 38: RAN4 should consider the RAN1 agreed parameters for the 6G sensing study. [Xiaomi]

- Recommended WF:

  - For the coexistence scenarios for 6G TRP mono-static, propose to consider the following cases:

| - Coexistence scenario | Aggressor signal | Victim signal | NOTE |
| --- | --- | --- | --- |
| 1A | Legacy NR DL | 6G ISAC BS Rx | Novel and prioritized |
| 2A | 6GR DL | 6G ISAC BS Rx | Case 2A is expected to be similar as Case 1A |
| 3A | 6G ISAC BS Tx | Legacy NR DL |  |
| 4A | 6G ISAC BS Tx | 6GR DL | Case 4A is expected to be similar as Case 3A |
| 5A | 6G ISAC BS Tx | 6G ISAC BS Rx |  |

  - Propose to discuss  whether both TDM-based (i.e., Half-duplex) or Full-duplex based monostatic sensing or only one mode will be considered during the RAN4 ISAC coexistence study. [Qualcomm]

  - Propose to consider  the self-interference/intra-site and inter-site co-channel interference (multiple BSs) and follow the decision from the 6G sensing feasibility study outcome.

  - Propose to discuss the signal type for different operation mode for TRP mono-static:

    - Single pulse as baseline  [Ericsson]

    - Single Pulse for TDM operation and contiguous wave for Full duplex operation mode;

    - Others

  - For TRP’s simulation assumption, discuss the following proposals for the initial calibration:

    - For 4GHz if agreeable,  discuss the following TRP antenna configuration:

- Implementation option 1 [TDM]

  - Tx: (12,8, 2,1,1;4,8)

  - Rx: (12,8, 2,1,1;4,8)

- Implementation option 2 [Full duplex]

  - Tx: (6,8, 2,1,1;2,8)

  - Rx: (6,8, 2,1,1;2,8)

    - For 7GHz  if agreeable,  discuss the following TRP antenna configuration:

- Implementation option 1  [TDM]

  - Tx: (24,16, 2,1,1;8,16)

  - Rx: (24,16, 2,1,1;8,16)

- Implementation option 2   [Full duplex]

  - Tx: (12,16, 2,1,1;4,16)

  - Rx: (12,16, 2,1,1;4,16)

    - For 30GHz if agreeable, discuss the following TRP antenna configuration:

- For 30GHz

- Implementation option 1 [TDM]

  - Tx: (12,8, 2,1,1;4,8)

  - Rx: (12,8, 2,1,1;4,8)

- Implementation option 2 [Full duplex]

  - Tx: (6,8, 2,1,1;2,8)

  - Rx: (6,8, 2,1,1;2,8)

  - For large-scale channel modelling,  propose to consider the following proposal

| Parameters | Values |
| --- | --- |
| Coupling loss for target channel | power scaling factor (pathloss, shadow fading, and RCS component σM included): LTX−SPST−RX=PLdBd1+PLdBd2+10lgc24πf2−10lgσRCS,A+SFdB,1+SFdB,2 |
| STX/SRX selection | Best N = 4 STX-SRX pairs to be selected for the target. see note 1 |
| Metrics | Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) see note 2 |
| NOTE 1: Based on the STX-SRX pairs with the smallest power scaling factor of the target channel. NOTE 2: CDFs can be separately generated for target channel, background channel. |  |

  - Where σRCS,A (σRCS=σMσDσS). is captured in TR 38.901 as following:

| Sensing target | 10lgσM (dBsm) | σσS_dB (dB) |
| --- | --- | --- |
| UAV with small size | -12.81 | 3.74 |

  - For small-scale channel modelling , further discuss the following proposals:

    - The TR 38.901 fast fading parameters are required for the multipath correction of estimation accuracy metrics. The small-scale parameters can be sampled from their log-normal distributions defined in TR 38.901 Table 7.5-6. The mean and standard deviation values are scenario-dependent (e.g., UMa-AV). Each drop independently generates a set of small-scale parameters. [Xiaomi]

  - Other simulation assumptions for 6G sensing for the initial calibration :

The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Scenario; UMa, SMa, FFS for RMa. Carrier frequency; [4 GHz, 7GHz and 30GHz]. System bandwidth; 100 MHz for 4GHz and 7GHz 200MHz for 30GHz. Numerology; SCS = 30kHz for 4GHz and 7GHz SCS=120kHz for 30GHz. BS Layout; 19 sites with 57 sectors. Inter-BS (2D) distance; Uma-AV: 500m for FR1 Uma-AV: 200m for FR2. BS antenna height; 25m for UMa-AV. Wrap-round; wrap-round. Note1: target(s) are dropped only in the center site, and inter-BS interference is not modelled. The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Target type; UAV with small size (0.3m x 0.4m x 0.2m). Target distribution when target(s) are dropped; N = 5 targets per sector in the center site. Optional: N is uniformly distributed from 1 to 10. Horizontal plane: uniformly distributed in a sector Vertical plane: Uniformly distributed between 25m and 300m, optionally distributed between 1.5m and 300m. Mobility; horizontal speed: uniformly distributed between 0 and 180km/h vertical speed: 0km/h. Note: N=0 will be discussed in a later proposal in relation to false alarm. The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Minimum BS-target 3D distance; 35 m. Minimum target-target (3D) distance; 10 m. Target outdoor/indoor proportion; 100% outdoor. LOS/NLOS; LOS and NLOS. Orientation; Random in horizontal domain. The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. BS antenna radiation pattern; clause 7.1.3 as captured in TR 38.922. BS antenna mechanic tilt; Without mechanical tilt for UAV detection. BS antenna sub-array fixed tilt; 3 degree per sub-array. When Tx/Rx operates simultaneously (baseline case), the following parameters are used for the evaluation on FR1 NR ISAC: Antenna isolation for the implementation option 2; - Option 1: 65dB - Option 2: 80dB. Maximum BS Tx power for FR1; - Option 1: 37dBm for the implementation option 2 [Full duplex] - Option 2: 52dBm for the implementation option 1[TDM] The above options are calculated with BS_maxpower = BS Rx saturation power + antenna isolation by assuming the BS Rx saturation power = -28dBm and the antenna isolation = 65dB and 80dB, respectively. - Optionally: companies should report the maximum BS Tx power when it is assumed that Tx and Rx don’t operate simultaneously. Companies should report how the maximum BS Tx power is derived. Antenna isolation for the implementation option 2; - Option 1: TBD - Option 2: TBD. Maximum BS Tx power for FR2; - Option 1: 30dBm for the implementation option 2 [Full duplex] - Option 2: 30Bm for the implementation option 1[TDM] The above options are calculated with BS_maxpower = BS Rx saturation power + antenna isolation by assuming the BS Rx saturation power = -28dBm and the antenna isolation = 65dB and 80dB, respectively. - Agreement When Tx/Rx operates simultaneously, the following evaluation parameters are agreed for the evaluation on NR ISAC. Self-interference; The residual leakage interference/noise is modelled as X dB below the noise floor.ce. The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Co-site inter-sector interference; Assumed to be same as intra-sector. Adjacent channel interference; Flat ACIR modelling. BS receiver noise figure; 5dB for 4GHz 6dB for 7GHz 10dB for 30dB.

  - For 5G/6G simulation assumption, propose to consider the following proposals:

    - For 4GHz if agreeable,  propose to use the simulation assumption as captured in TR 38.922 for 4GHz.

    - For 7GHz if agreeable, propose to use the simulation assumption as captured in TR 38.922 for 7125-8400MHz

    - For 30GHz if agreeable, propose to use the simulation assumption as captured in TR 38.858 for FR2-1.

    - NOTE: For 6GR 7GHz, it might be updated according to the agreement reached in 6GR coexistence study thread.

  - Network layout for aggressor and victim network:

    - 19 sites with 57 sectors with the wrap round

    - Grid shift for overlapping coverage: 0%, 10%, 50% and 100%

    - FFS for the isolation between rural area between aggressor and victim

  - For performance metric for 6GR and 5GR as victim, use the use the legacy throughput loss as basic metric.

  - For performance metric for 6G sensing, discuss the following metric and any down selection for the following metrics:

    - Detection Probability

    - False Alarm Probability

    - Horizontal Localization Accuracy

    - Vertical Localization Accuracy

    - Velocity Accuracy

Issue 1-5: Simulation methodology for TRP mono-static

- Proposal 1: Discuss the following simulation procedures for TRP mono-static. [Xiaomi]

a) Per-drop calculation phase

The following steps are executed for each Monte Carlo drop. The total number of drops is 1000 per simulation run.

Step 1: Topology generation

            - Drop Ntarget  targets uniformly within each sector of the center site.

            - For each target, sample the height from a uniform distribution over [25 m, 300 m].

            - For each target, sample the horizontal speed from a uniform distribution over [0, 180] km/h.

            - Record the true position (x, y, z) and velocity (vx, vy, vz) of each target.

            - Ensure minimum 3D distance between any target and any BS is at least 10 m, and between any pair of targets is at least 10 m.

Step 2: Coupling loss calculation

            - For each target-sensing node pair, calculate the 3D distances dTx−target and dtarget−Rx.

            - Determine LOS/NLOS condition using the LOS probability model from TR 38.901 Section 7.4.2.

            - Calculate the path loss PL(d) for each leg using the UMa path loss model

            - Generate independent shadow fading values SF1 and SF2 from log-normal distributions.

            - Compute the coupling loss L_coupling per the formula.

            - Select the best N = 4 Tx-Rx pairs with the smallest coupling loss for each target.

Step 3: Interference power calculation

            - For each interferer, calculate the 3D distance dintf from the interferer to the victim sensing receiver.

            - Calculate the interference path loss PLintfdintf using the same propagation model.

            - Calculate the antenna gain Gintf  from the interferer towards the victim receiver.

            - Determine the ACIR value based on the configured ACLR and ACS.

            - Calculate the time-frequency overlap ratio rℎoTF  between the sensing resource and the interferer resource.

            - Compute the interference power contribution: Pintf,i = Ptx,intf − PLintf + Gintf − ACIR + 10∗log10(rℎoTF).

            - Aggregate all interferers: Pinterference = 10∗log10sumi10Pintf,i10.

Step 4: SINR computation

            - Calculate the target echo power: Psignal = Ptx − Lcoupling + Gtx + Grx.

            - Calculate the noise power: Pnoise = −174 + 10∗log10(BW) + NF.

            - Calculate the total noise-plus-interference: Ptotal = 10∗log10(10Pnoise10 + 10Pinterference10).

            - Compute the sensing SINR: SINR = Psignal − Ptotal [dB].

            - Compute the coherent integration gain: Gcoℎerent = 10∗log10(Nsubcarrier ∗ Nsymbol).

            - Compute the effective SINR: SINReff = SINR + GcoℎerentdB.

Step 5: Small-scale parameter extraction

The TR 38.901 fast fading parameters are required for the multipath correction of estimation accuracy metrics. The small-scale parameters can be sampled from their log-normal distributions defined in TR 38.901 Table 7.5-6. The mean and standard deviation values are scenario-dependent (e.g., UMa-AV). Each drop independently generates a set of small-scale parameters.

Step 6: Performance metric computation

For each target, the following five performance metrics are computed using the models

Statistical analysis phase

After completing all Monte Carlo drops, the following statistical analysis is performed:

Result collection

Collect all per-drop, per-target metric values into vectors:

Table 3: Collected metric data structures

| Metric | Data structure | Dimension |
| --- | --- | --- |
| Pmd | Per-target missed detection probability | Ndrops x Ntargets |
| PfaType 1 | Per-drop false alarm (no target drops) | Ndrops_no_target |
| PfaType 2 | Per-drop false alarm (with target drops) | Ndrops_witℎ_target |
| sigmaH | Per-target horizontal positioning error | Ndrops x Ntargets |
| sigmaV | Per-target vertical positioning error | Ndrops x Ntargets |
| sigmav | Per-target radial velocity error | Ndrops x Ntargets |

CDF statistics and KPI compliance check

For each metric, compute the Cumulative Distribution Function (CDF) and extract key percentile values:

            - [5th percentile (representing cell-edge / worst-case performance)]

            - [50th percentile (median performance)]

            - [90th percentile (representing typical performance)]

The CDF curves for each metric are generated for both the baseline (no interference) and the interfered cases.

- Proposal 2: Until a verified mapping between SINR_sensing and sensing performance is established, RAN4 could adopt the following interim criterion for coexistence evaluation: A degradation of SINR_sensing of no more than X dB relative to the interference-free case is considered acceptable. [Xiaomi]

- Proposal 3: To consider following equations to convert the SINR from RAN4 co-ex simulator to sensing SINR, which can be used to derive SINR performance later. [Vivo]

SINR_sensing [dB] = SINR_RAN4_coex [dB] + 10·log₁₀(N_sc) + 10·log₁₀(N_sym)

where

- SINR_RAN4_coex	is the SINR (dB) obtained from legacy RAN4 coexistence analysis, which assumed as flat across all subcarriers,

- N_sc						is the number of subcarriers allocated to the sensing signal within one OFDM symbol,

- N_sym					is the number of OFDM symbols.

SINR_RAN4_coex [dB] = S_wanted,sc [dBm] − (I_total,sc [dBm] ⊕ N_o,sc [dBm])

where ⊕ denotes power summation in linear domain, and:

- S_wanted,sc is the wanted signal power per subcarrier at the sensing receiver input, given by:

S_wanted,sc [dBm] = P_Tx_sensing [dBm] − PL [dB] − L_reflect [dB] − 10·log₁₀(N_sc) – 10·log₁₀(N_sym)

  - P_Tx_sensing is the total conducted power of the sensing transmitter over the allocated bandwidth.

  - PL is the one-way large-scale path loss (including shadow fading) between the sensing transmitter and the target, and between the target and the sensing receiver, i.e. the round-trip propagation loss.

  - L_reflect is the reflection loss  accounting for the fact that only a fraction of the incident energy is scattered back towards the receiver.

  - The term − 10·log₁₀(N_sc) – 10·log₁₀(N_sym) converts the total received wanted signal power (after round-trip propagation and reflection) to a per-subcarrier power, consistent with the per-subcarrier definition of SINR_RAN4_coex.

- I_total,sc is the total interference power per subcarrier at the sensing receiver input, consisting of Intra-system interference and Inter-system interference.

- N_o,sc is the thermal noise power per subcarrier: N_sc [dBm] = N₀ [dBm/Hz] + 10·log₁₀(SCS), where N₀ is the thermal noise power spectral density and SCS is the subcarrier spacing.

- Proposal 4: To satisfy the 1m/s velocity resolution requirement for UAV targets in FR1 bands, the coherent accumulation time for velocity measurement simulations should be carefully configured. As shown by the comparative baseline analysis (which requires >77ms at lower frequencies), sufficient coherent accumulation time should be assumed to ensure reliable velocity estimation across different FR1 scenarios. [BUPT, China telecom]

- Proposal 5: When designing ISAC frame structures and reference signal patterns, an equidistant distributed symbol arrangement in the time domain should be assumed, as it is more beneficial for velocity measurement than a centralized symbol arrangement. [BUPT, China telecom]

- Proposal 6: RAN4 to discuss the pros and cons of approximation framework based on theoretical closed form expressions that could be used to quantify impact of adjacent channel interference on the sensing performance. [Qualcomm]

- Proposal 7 Single pulse as baseline to compute SINR [Ericsson]

- Proposal 8 The simulation results should compare the SINR CDFs with and without interference  [Ericsson]

- Proposal 9 Sensitivity analysis for detection and tracking accuracy considering number of pulses and pulse integration for different interference level low/high situations [Ericsson]

- Proposal 10 Detection probability versus SINR is needed to define acceptable SINR threshold value  [Ericsson]

- Recommended WF:

  - Collect the views for the above proposals;

Issue 1-6: Simulation assumptions for TRP bi-static

- Proposal 1: For the TRP bi-static, the coexistence studies between 6G sensing and the legacy system need to be conducted to figure out the appropriate RF requirements.  [ZTE]

| Case | Aggressor | Aggressor baseline | Victim | Remarks |
| --- | --- | --- | --- | --- |
| 4 | ISAC | TDD DL | TDD DL | Observation 1: For 6G sensing TRP interfering 6GR communication DL, if the sensing TRP can comply with 6GR BS requirements, then the coexistence performance between 6G sensing and 6G communication or 5G communication can be guaranteed. |
| 5 | TDD DL | No aggressor network | ISAC | Observation 2: For 6GR communication DL interfering 6GR sensing, the BS2BS CLI including reflected path and non-reflected path should be considered for the TRP bi-static. |
| 6 | ISAC | No aggressor network | ISAC | Observation 3: For 6GR sensing interfering 6GR sensing, the BS2BS CLI should be considered for the TRP bi-static. |
| NOTE: assume the ISAC to be deployed in the DL symbols to ensure the coverage as 1st priority |  |  |  |  |

- Proposal 2: for the simulation assumption for UAV sensing evaluation in RAN4, propose to use the same simulation assumption as 6GR mono-static sensing mode with the implementation option 1 [TDM manner]. [ZTE]

- Proposal 3: for the simulation methodology for 6G TRP with bi-static sensing mode, propose to use the same simulation methodology as 6GR mono-static sensing mode. [ZTE]

- Proposal 4 Use for the 4 GHz UAV and BS-to-BS bistatic and BS monostatic the parameters from the Table 2.2.1-1a to Table 2.2.1-1e and Table 2.2.2-1b [Ericsson]

- Table 2.3.1-1a Network layout and system parameters

| Parameters | Value |
| --- | --- |
| Frequency and bandwidth | 4 GHz (covering the 3-4 GHz range) with 100 MHz bandwidth |
| Network layout | UMa/SMa: hexagonal grid, 19 macro sites, 3 sectors per site with wrap around (or victim centre sector) |
| Inter-site distance | 0.45 km (urban) and 0.9 km (suburban) Table 4.2.1.1-1 in TR 38.921 |
| Multi operator’s layout | UMa/SMa: 0%, 10%, 50% and 100% grid shift |
| TDD DL/UL ratio | 80/20% |
| Synchronised/unsynchronised TDD operation in multi-operator context | Compatible frame structure to align uplink/downlink switching points. ISAC can use DL time slot for transmitting and receiving sensing signals |
| Cell load | UMa/SMa interferer: 10%, 50% and full buffer |

- Table 2.3.1-1b BS parameters (mainly from Section 4.2 and 4.4 in TR 38.922)

| Parameters | Value |
| --- | --- |
| BS power TRP | 49, 53 dBm |
| BS antenna height | UMa/SMa: 25 m |
| Antenna mechanical tilt | UMa/SMa: 6 degrees downtilt |
| AAS antenna elements | Table 4.4.1.2-1 in Reference TR 38.922 (for starting point) |
| ACLR/ACS | 45/46 dB |

Table 2.3.1-1c UE parameters (mainly from Section 4.3 in TR 38.922)

| UE Parameters | Value |
| --- | --- |
| Outdoor/indoor | Outdoor and indoor with 20/80% |
| Antenna | 1Tx/2Rx |
| Antenna gain | Isotropic: 0 dBi |
| Max Tx power | 23 dBm |
| Power control | Section 9.1 TR 36.942 |
| SNR target | 15 dB |
| Noise figure | 9 dB |
| Antenna height | 1.5 m for outdoor |
| Distribution (horizontal) | Uniform |
| Minimum BS - UE distance (2D) | 35 m For Uncoordinated deployment, minimum distance applies also to UEs and BS across different deployments. |

Table 2.3.1-1d UAV parameters

| Parameters | Value |
| --- | --- |
| UAV distribution | Uniform distribution with heights 25 to 300 meters (TR 38.901 / TR 36.777) |
| Minimum BS - UAV distance | 10 meters 3D (TR 38.901 / TR 36.777) |

Table 2.3.1-1e Channel models

| Parameters | Value |
| --- | --- |
| BS-UE channel model | UMa (Table 7.4.1-1 in TR 38.901) |
| BS-BS channel model | UMa TR 38.901. LoS model is the default defined for UMa For BS-to-BS interference Line of Sight (LoS) probability with 100% or FSPL |
| BS-UAV channel model | Rel-19 extensions for ISAC in TR 38.901 - Focusing on large scale parameters - RCS of the sensing target, the model captured in clause 7.9.2.1 |

- Proposal 5 Single pulse as baseline to compute SINR [Ericsson]

- Proposal 6 The simulation results should compare the SINR CDFs with and without interference  [Ericsson]

- Proposal 7 Sensitivity analysis for detection and tracking accuracy considering number of pulses and pulse integration for different interference level low/high situations [Ericsson]

- Proposal 8 Detection probability versus SINR is needed to define acceptable SINR threshold value  [Ericsson]

- Recommended WF:

  - For the coexistence scenarios for 6G TRP bi-static, propose to consider the following cases:

| - Coexistence scenario | Aggressor signal | Victim signal | NOTE |
| --- | --- | --- | --- |
| 1B | Legacy NR DL | 6G ISAC BS Rx | Novel and prioritized |
| 2B | 6GR DL | 6G ISAC BS Rx | Case 2B is expected to be similar as Case 1B |
| 3B | 6G ISAC BS Tx | Legacy NR DL |  |
| 4B | 6G ISAC BS Tx | 6GR DL | Case 4A is expected to be similar as Case 3A |
| 5B | 6G ISAC BS Tx | 6G ISAC BS Rx |  |

  - For the simulation assumption for TRP bi-static, propose to discuss the following proposals:

    - Propose to use the same simulation assumption as TRP mono-static sensing mode with the implementation option 1 [TDM manner]. [ZTE]

    - For the other simulation assumptions, follow the agreement as reached for TRP mono-static sensing mode;

Issue 1-6: Simulation assumptions for TRP-UE bi-static

- Proposal 1: For coexistence study in RAN4, sensing mode of aNB-UE bi-static is considered and starts with the following coexistence cases, while case (b) need to be further studied and determine the necessity. [Huawei]

| Sensing mode and sensing target | index | Aggressor | victim |
| --- | --- | --- | --- |
| Vehicles, TRP-UE bistatic | a | Target reflection | NR DL |
|  | b | Sensing signal | NR DL |
|  | c | NR DL | Target reflection |

- Proposal 2: Before simulation evaluation, RAN4 need to further analyse case (a) with target reflection as the aggressor in scenario#1 & #2 on the reflection power level. [Huawei]

- Proposal 3: To confirm TRP-UE/UE-TRP bistatic mode would not introduce additional interference to legacy system from sensing operation. [Vivo]

- Proposal 4: For the TRP-UE bi-static, the coexistence studies between 6G sensing and the legacy system need to be conducted to figure out the appropriate RF requirements.  [ZTE]

Table 2.4.1-2: Adjacent channel co-existence cases for TRP-UE bi-static deployment

| Case | Aggressor | Aggressor baseline | Victim | Remarks |
| --- | --- | --- | --- | --- |
| 4 | ISAC | TDD DL | TDD DL | Observation 1: For 6G sensing TRP interfering 6GR communication in DL, if the sensing TRP can comply with 6GR BS requirements, then the coexistence performance between 6G sensing and 6G communication or 5G communication can be guaranteed. |
| 5 | TDD DL | No aggressor network | ISAC | Observation 2: For 6GR communication DL interfering 6GR sensing, the BS2UE ACI should be considered for TRP-UE bi-static sensing mode. |
| 6 | ISAC | No aggressor network | ISAC | Observation 3: For 6GR sensing interfering 6GR sensing, the BS2UE ACI should be considered for the TRP-UE bi-static sensing mode. |
| NOTE: assume the ISAC to be deployed in the DL symbols to ensure the coverage as 1st priority |  |  |  |  |

- Proposal 5: for the simulation assumption for TRP-UE sensing evaluation in RAN4, propose to use the same simulation assumption as 6GR mono-static sensing mode with the implementation option 1 [TDM manner]. [ZTE]

- Proposal 6: for the simulation methodology for 6G TRP-UE with bi-static sensing mode, propose to use the same simulation methodology as 6GR mono-static sensing mode [ZTE]

- Recommended WF:

  - For the coexistence scenarios for 6G TRP-UE bi-static, propose to consider the following cases:

| - Coexistence scenario | Aggressor signal | Victim signal | NOTE |
| --- | --- | --- | --- |
| 1C | Legacy NR DL | 6G ISAC UE Rx | Novel and prioritized |
| 2C | 6GR DL | 6G ISAC UE Rx | Case 2C is expected to be similar as Case 1C |
| 3C | 6G ISAC BS Tx | Legacy NR DL |  |
| 4C | 6G ISAC BS Tx | 6GR DL | Case 4C is expected to be similar as Case 3C |
| 5C | 6G ISAC BS Tx | 6G ISAC BS Rx |  |

  - Further discuss the impacts of target reflection signal towards the victim DL [Huawei]

  - Further discuss whether Coexistence scenario 3C and 4C could be skipped if ISAC BS is complying with IMT BS emission mask and ACLR requirements;  [Vivo]

  - For the simulation assumption for TRP-UE bi-static, propose to discuss the following proposals:

    - Propose to use the same simulation assumption as TRP mono-static sensing mode with the implementation option 1 [TDM manner]. [ZTE]

    - For RCS value for the vehicle, propose to consider the following value:

Table 7.9.2.1-4: Parameters on RCS for vehicle with single scattering point

|  | 10lgσMσD (dBsm) |  |  |  |  |  |  |  | 10lgσM (dBsm) | σσS_dB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ϕcenter in [°] | ϕ3dB in [°] | θcenter in [°] | θ3dB in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |  |  |
| Left | 90 | 26.90 | 79.70 | 44.42 | 20.75 | 13.68 | [30,180] | [45,135) | 11.25 | 3.41 |
| Back | 180 | 36.32 | 79.65 | 36.73 | 14.56 | 7.50 | [30,180] | [135,225) |  |  |
| Right | 270 | 26.90 | 79.70 | 44.42 | 20.75 | 13.68 | [30,180] | [225,315) |  |  |
| Front | 0 | 40.54 | 71.75 | 29.13 | 15.52 | 8.45 | [30,180] | [-45, 45) |  |  |
| Roof | - | - | 0.00 | 18.13 | 21.26 | 14.19 | [0,30) | [0,360) |  |  |
| NOTE: When θ is in the range [0, 30), σHdB ϕ=0. |  |  |  |  |  |  |  |  |  |  |

    - For the network layout of highway and Urban grid, propose to consider the following proposals:

      - Highway (as defined in the A.1.2 of TR 36.885)

      - Urban grid  (as defined in the A.1.2 of TR 36.885)

    - Further discuss the ISAC UE in the sensing mode and further discuss the following UE types:

      - Vehicle UE

      - RSU

      - Pedestrian UE

    - For the other simulation assumptions, follow the agreement as reached for TRP mono-static sensing mode;

Issue 1-7: Simulation assumptions for UE-TRP bi-static

- Proposal 1: To confirm TRP-UE/UE-TRP bistatic mode would not introduce additional interference to legacy system from sensing operation. [Vivo]

- Proposal 2: For the UE-TRP bi-static, the coexistence studies between 6G sensing and the legacy system need to be conducted to figure out the appropriate RF requirements.  [ZTE]

Table 2.4.1-2: Adjacent channel co-existence cases for TRP bi-static deployment

| Case | Aggressor | Aggressor baseline | Victim | Remarks |
| --- | --- | --- | --- | --- |
| 4 | ISAC | TDD DL | TDD DL | Observation 1: For 6G sensing TRP interfering 6GR communication in DL, if the sensing TRP can comply with 6GR UE requirements, then the coexistence performance between 6G sensing and 6G communication or 5G communication can be guaranteed. |
| 5 | TDD DL | No aggressor network | ISAC | Observation 2: For 6GR communication UL interfering 6GR UE-TRP bi-static sensing mode, the UE2BS ACI should be considered. |
| 6 | ISAC | No aggressor network | ISAC | Observation 3: For 6GR sensing UL interfering 6GR UE-TRP bi-static sensing mode, the UE2BS ACI should be considered. |
| NOTE: assume the ISAC to be deployed in the DL symbols to ensure the coverage as 1st priority |  |  |  |  |

- Proposal 3: for the simulation assumption for UE-TRP bi-static sensing evaluation in RAN4, propose to use the same simulation assumption as 6GR mono-static sensing mode with the implementation option 1 [TDM manner]. [ZTE]

- Proposal 4: for the simulation methodology for 6G UE-TRP with bi-static sensing mode, propose to use the same simulation methodology as 6GR mono-static sensing mode. [ZTE]

- Recommended WF:

  - For the coexistence scenarios for 6G UE-TRP bi-static, propose to consider the following cases:

| - Coexistence scenario | Aggressor signal | Victim signal | NOTE |
| --- | --- | --- | --- |
| 1D | Legacy NR UL | 6G ISAC BS Rx | Novel and prioritized |
| 2D | 6GR UL | 6G ISAC BS Rx | Case 2D is expected to be similar as Case 1D |
| 3D | 6G ISAC UE Tx | Legacy NR UL |  |
| 4D | 6G ISAC UE Tx | 6GR UL | Case 4D is expected to be similar as Case 3D |
| 5D | 6G ISAC UE Tx | 6G ISAC BS Rx |  |

  - Further discuss whether Coexistence scenario 3D and 4D could be skipped if ISAC BS is complying with IMT BS emission mask and ACLR requirements;  [Vivo]

  - For the simulation assumption for TRP-UE bi-static, propose to discuss the following proposals:

    - Propose to use the same simulation assumption as TRP mono-static sensing mode with the implementation option 1 [TDM manner]. [ZTE]

    - For RCS value for the vehicle, propose to consider the following value:

Table 7.9.2.1-4: Parameters on RCS for vehicle with single scattering point

|  | 10lgσMσD (dBsm) |  |  |  |  |  |  |  | 10lgσM (dBsm) | σσS_dB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ϕcenter in [°] | ϕ3dB in [°] | θcenter in [°] | θ3dB in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |  |  |
| Left | 90 | 26.90 | 79.70 | 44.42 | 20.75 | 13.68 | [30,180] | [45,135) | 11.25 | 3.41 |
| Back | 180 | 36.32 | 79.65 | 36.73 | 14.56 | 7.50 | [30,180] | [135,225) |  |  |
| Right | 270 | 26.90 | 79.70 | 44.42 | 20.75 | 13.68 | [30,180] | [225,315) |  |  |
| Front | 0 | 40.54 | 71.75 | 29.13 | 15.52 | 8.45 | [30,180] | [-45, 45) |  |  |
| Roof | - | - | 0.00 | 18.13 | 21.26 | 14.19 | [0,30) | [0,360) |  |  |
| NOTE: When θ is in the range [0, 30), σHdB ϕ=0. |  |  |  |  |  |  |  |  |  |  |

    - For the network layout of highway and Urban grid, propose to consider the following proposals:

      - Highway (as defined in the A.1.2 of TR 36.885)

      - Urban grid  (as defined in the A.1.2 of TR 36.885)

    - Further discuss the ISAC UE in the sensing mode and further discuss the following UE types:

      - Vehicle UE

      - RSU

      - Pedestrian UE

    - For the other simulation assumptions, follow the agreement as reached for TRP mono-static sensing mode;

- 2.2. Summary for impact on the existing requirements and potential new requirements

Issue 2-1: General for 6G sensing requirements

- Proposal 1: RAN4 to assess UE RF impacts once RAN1 reaches sufficient agreements.  [Nokia]

- Proposal 2: RAN4 to study the RAN4 impact on BS RF requirements once RAN1 makes sufficient agreements. [Nokia]

- Proposal 3: It is proposed to consider also BS type 1-H for 6G sensing. [Nokia]

- Proposal 4: RAN4 can discuss whether different sensing BS classes are needed for the different rated output power (e.g. 37dBm or 52dBm), after RAN4 can check the link budget firstly according to the Radar Function. [CATT]

- Proposal 5: at least the lower than 1GHz frequency is not feasible for sensing. band s79, around 7GHz and band s258 are suggested as example bands. [CMCC]

- Proposal 6: CBW definition is subject to the distance resolution. the larger CBW, the higher distance resolution.

- Proposal 7: RAN4 should consider that the RF requirement of 6G communication should be baseline and not impacted by sensing operation.  [Vivo]

- Proposal 8: RAN4 should further discuss how to specify a per-UE RF requirements to meet the 6G sensing KPI, coordination and clarification from RAN1 about this aspect may be needed. [Vivo]

- Recommended WF:

  - Discuss whether BS type 1-H could be studied for 6G ISAC BS:

      - Supported by [Samsung, ZTE and Nokia]

  - Discuss the applicable frequency range for 6G sensing operation:

    - Option 1: at least above 1GHz  [CMCC]

    - Option 2: to be discussed during the WI phase;

    - Option 3: others

  - Discuss whether to define different BS class if maximum output power is different for TRP mono-static sensing mode [CATT]

  - Discuss whether the RF requirement for the communication link would be impacted or not due to sensing operation:

    - Option 1: Not impacted by sensing operation  [Vivo]

    - Option 2: Further discuss these issue .e.g. (simultaneous Uu link and sensing transmission at the different bands etc)

  - Discuss how to define the per-UE requirement if the sensing results are processed based on the multiple UE’s measurement results etc.  [Vivo]

Issue 2-2: The impacts on the existing requirements

- Proposal 1: Existing requirements need to be studied and evaluated for the coexistence between 6G sensing and NR/6G communication, considering new sensing performance metrics. [Huawei]

- Proposal 2: For the impacts on the existing RF requirement, propose to discuss the sensing mode case by case. [ZTE]

| Sensing mode | Tx | Rx |
| --- | --- | --- |
| TRP mono-static | TRP Tx requirement is needed | TRP Rx requirement is needed |
| TRP bi-static | TRP Tx requirement is needed | TRP Rx requirement is needed |
| TRP-UE bi-static | TRP Tx requirement is needed | UE Rx requirement is needed |
| UE-TRP bi-static | UE Tx requirement is needed | TRP Rx requirement is needed |
| UE monostatic | UE Tx requirement is needed | UE Rx requirement is needed |
| UE bi-static | UE Tx requirement is needed | UE Rx requirement is needed |

- Proposal 3: For the TRP mono-static sensing mode, propose to consider the following RF requirements for TRP mono-static sensing mode;  [ZTE]

| RF requirements | 6GR BS | 6G ISAC |
| --- | --- | --- |
| carrier bandwidth | Yes | The same as 6GR BS |
| Numerology | Yes | FFS |
| Channel raster | Yes | ## FFS |
| Sync raster | Yes | ## N/A |
| Base station output power | Yes | ## Accuracy is expected to be the same as 6GR BS |
| Output power dynamics | Yes | N/A |
| Transmit ON/OFF power | Yes | ## FFS ## For full duplex sensing TRP solution, it might be slightly different compared with TDD BS. |
| OBW | Yes | ## The same as 6GR BS |
| Tx intermodulation | Yes | ## The same as 6GR BS |
| ACLR | Yes | ## Expected to be the same as communication DL if DL spectrum is used for sensing transmission. |
| Operating band unwanted emissions | Yes | ## Expected to be the same as communication DL if DL spectrum is used for sensing transmission. |
| Transmitter spurious emissions | Yes | The same as 6GR BS |
| Rx part |  |  |
| dynamic range | Yes | FFS |
| NBB | Yes | FFS |
| RX NB IMD | Yes | FFS |
| RX spurious emission | Yes | The same as 6GR BS |
| RX ICS | Yes | FFS |

- Proposal 4: RAN4 can consider the Transmit power, ACLR, SEM, EVM, phase noise, Transmit ON/OFF time mask as the beginning of UE Tx requirements, not precluding other requirements. [Xiaomi]

- Proposal 5: RAN4 can consider the Reference Sensitivity or sensing sensitivity, ACS, blocking characteristics, Intermodulation characteristic as the beginning of UE Rx requirements, not precluding other requirements. [Xiaomi]

- Recommended WF:

  - For the impacts on the existing requirements, propose to discuss the requirements for each sensing mode and use case;

  - Encourage companies to provide the views whether existing RF requirements could be reused or adjusted once sensing signal is transmitted either from TRP or UE side;

Issue 2-3: Potential new RF requirements

- Proposal 1: RAN4 to study and define synchronization requirements for 6G bistatic sensing. [Nokia]

- Proposal 2: RAN4 to study and define blind range requirements for 6G TRP monostatic sensing. [Nokia]

- Proposal 3: For aNB mono-static based UAV sensing scenario, the new requirements considered in RAN4 should include Tx-Rx switching time interval for pulse wave-based sensing, e.g. below 2.34μs, and sensing signal received sensitivity. [Huawei]

- Proposal 4: For aNB-UE/UE-aNB bi-static based vehicle sensing scenario, the new requirements considered in RAN4 should include UE phase continuity and sensing signal received sensitivity. [Huawei]

- Proposal 5: For new REFSENS requirements for sensing BS, the following two points at least should be studied, which need the progress of other WGs. [CATT]

  - Point 1: the mapping between SNR and detection probability.

  - Point 2: new “FRC” for testing REFSENS of sensing BS. The new “FRC” can be simulated by TE.

- Proposal 6: RAN4 can consider the Transmit power, ACLR, SEM, EVM, phase noise, Transmit ON/OFF time mask as the beginning of UE Tx requirements, not precluding other requirements. [Xiaomi]

- Proposal 7: RAN4 can consider the Reference Sensitivity or sensing sensitivity, ACS, blocking characteristics, Intermodulation characteristic as the beginning of UE Rx requirements, not precluding other requirements. [Xiaomi]

- Proposal 8: RAN4 should study a new sensitivity requirement to quantify sensing receiver performance with supporting two-dimensional coherent processing. [Vivo]

- Proposal 9: RAN4 should consider UE RF impairments when evaluate the sensing requirements. [Vivo]

- Proposal 10: RAN4 is suggested to focus on enhanced synchronization requirements for bi-static sensing mode. [CMCC]

- Proposal 11: it’s suggested to only focus on one or two sensing performance metrics instead of throughput loss when defining RF requirements for each use case. For example, for human use case, we can only use detection probability to define REFSENSE requirements and for UAV use case, we can use localization accuracy and velocity accuracy to define ACLR requirements.  [CMCC]

- Proposal 12: REFSENSE like requirements are still necessary. RAN4 need to find out the target SNR for REFSENSE definition based on the mapping between SNR and performance metric, e.g. the detection probability. [CMCC]

- Proposal 13:From the results of the coexistence studies RAN4 needs to decide whether new requirements are needed. [Ericsson]

- Proposal 14: For the potential new RF requirement for 6G sensing, propose to discuss the sensing mode case by case.  [ZTE]

| Sensing mode | Tx | Rx |
| --- | --- | --- |
| TRP mono-static | TRP Tx requirement is needed - Tx quality for sensing signal if needed; - Fast ON-OFF transition period for switching between transmission and reception of sensing slot if needed | TRP Rx requirement is needed - REFSENS requirements (e.g. miss detection/false alarm detection probability) for sensing signal - Other ACS/IBB blocking requirement due to the BS2BS CLI interference if exists |
| TRP bi-static | TRP Tx requirement is needed - Tx quality for sensing signal if needed; | TRP Rx requirement is needed - REFSENS requirements (e.g. miss detection/false alarm detection probability) for sensing signal - Other ACS/IBB blocking requirement due to the BS2BS CLI interference if exists - Tight timing sync requirement between Tx and Rx |
| TRP-UE bi-static | TRP Tx requirement is needed - Tx quality for sensing signal if needed; | UE Rx requirement is needed - REFSENS requirements (e.g. miss detection/false alarm detection probability) for sensing signal - ACS/IBB with the sensing signal as wanted signal with certain degradation - Potential large receiver dynamic range requirements |
| UE-TRP bi-static | UE Tx requirement is needed - Tx quality for sensing signal if needed; | TRP Rx requirement is needed - REFSENS requirements (e.g. miss detection/false alarm detection probability) for sensing signal - ACS/IBB with sensing signal as wanted signal with certain degradation - Potential large receiver dynamic range requirements |
| UE monostatic | UE Tx requirement is needed - Tx quality for sensing signal if needed; - Fast ON-OFF transition period for switching between transmission and reception of sensing slot if needed | UE Rx requirement is needed - REFSENS requirements (e.g. miss detection/false alarm detection probability) for sensing signal - Other ACS/IBB blocking requirement due to the UE2UE CLI interference if exists |
| UE bi-static | UE Tx requirement is needed - Tx quality for sensing signal if needed; | UE Rx requirement is needed - REFSENS requirements (e.g. miss detection/false alarm detection probability) for sensing signal - Other ACS/IBB blocking requirement due to the UE2UE CLI interference if exists |

- Recommended WF:

  - For the potential new requirements, propose to discuss for each sensing mode and use case;

  - For bi-static sensing mode including TRP-TRP, TRP-UE, UE-TRP and UE-UE, propose to discuss the enhanced sync performance between two nodes;

  - For mono-static sensing mode, propose to discuss Tx-Rx switching time interval for pulse wave-based sensing and Tx transmission time;

  - For the Tx and Rx requirements for sensing transmission and reception, further discuss the applicable requirements e.g.;

    - UE phase continuity for CPI coherent combining across different symbols or time locations;

    - EVM for sensing reference signal

    - Enhanced TDD ON-FF transition period for the minimum blind detection area

    - Metrics for sensing receiver (e.g. miss detection, false alarm ratio, horizontal/vertical accuracy, velocity Accuracy etc)

    - Other new requirements are not precluded.

    - NOTE Others implementation factors should be also considered when defining the requirements.

Issue 2-4: Potential new RRM requirements

- Proposal 1: RAN4 to discuss the RRM impact in 6G sensing considering the following components [Nokia]:

  - 	Configuration of sensing reference signal at the sensing transmitter (STx) and at the sensing receiver (SRx).

  - 	Sensing reference signal measurement procedure at the SRx.

  - 	Sensing measurement reporting by SRx towards the target network entity, in particular the Sensing Function.

- Proposal 2: Postpone the discussion for RRM requirement for 6G sensing until RAN4 finished the feasibility study for the corresponding sensing modes and use cases. [CATT]

- Proposal 3:  In RAN4 the corresponding requirements for the measurement reporting from UE and TRP are desired. But how to define the exact requirements is still upon RAN1’s further conclusion on the measurement reporting mechanism.  [Xiaomi]

- Proposal 4: RAN4 can study the potential RRM requirements impact (e.g. cell reselection, handover, beam management) because of the sensing assisted mechanism.  [Xiaomi]

- Proposal 5: UE RRM requirements can start with the study on the measurement quantities including delay, Doppler frequency, angle and power. [Vivo]

- Proposal 6: Postpone the RAN4 study of sensing related RRM until other WGs have sufficient progress/conclusions.  [CMCC]

- Proposal 7: RAN4 considers the following in its RRM discussions for ISAC: [Ericsson]

  - -	Not all ISAC deployment scenarios will have the same RAN4 RRM specification impact;

  - -	Some ISAC measurement and accuracy requirements may need to be specified by RAN4, since new sensing-specific measurements may be introduced;

  - -	Not only the requirements for RRM sensing measurements may be needed, but also the potential impact on other RRM requirements ensuring the general RRM performance (e.g., for other non-sensing UE measurements and procedures) needs to be considered for UE performing sensing measurements and/or transmitting radio signals for sensing.

- Proposal 8: RAN4 considers studying CDFs of received signal power levels and SINRs of the sensing signals, which may impact side conditions in the requirements. [Ericsson]

- Proposal 9: Consider UE-to-UE interference in UL resources when discussing UE monostatic sensing. [Ericsson]

- Proposal 10: Consider the sensing continuity requirement impact on the RAN4 assumptions, requirements, and the measurement procedure for sensing. [Ericsson]

- Proposal 11: For the communication RRM impacts due to 6G sensing measurement, propose to postpone the discussion until there are sufficient progress made in RAN1/RAN2/RAN3. [ZTE]

- Proposal 12: In addition to measurement requirements for UE, RAN4 further consider whether the measurement requirements for aNB/TRP should be specified as well. [CMCC]

- Recommended WF:

  - For the configuration, measurement metric and measurement reporting for 6G sensing, RAN4 need to take these factors agreed in RAN1/2 into account when defining the RRM requirements;

  - For the measurement quantities or report range in the RRM specification, propose to postpone the discussion until measurement metrics are agreed in RAN1/2.

  - RAN4 considers studying CDFs of received signal power levels and SINRs of the sensing signals, which may impact side conditions in the requirements.

  - For sensing assisted mobility as captured in 6G TR 38.914, discuss how it would impact on the RRM requirements and also coordinate with other WGs.

  - For sensing continuity as agreed by SA2, discuss how the mobility would impact on sensing continuity and coordinate with other WGs.

  - For UE-to-UE interference in UL resources in the UE mono-static and UE bi-static, propose to discuss its RRM impacts;

  - Discus whether to define the measurement report relevant requirements for aNB/TRP

- 2.3. Summary for Testing aspects

Issue 3-1:General  6G sensing conformance testing

- Proposal 1: Postpone the test discussion until the related RF requirements and test cases are clarified. [Huawei]

- Observation 1: BS type support may differ between mono-static sensing and bi-static sensing. [Ericsson]

- Proposal 5: First agree on ISAC BS RF core requirements with respect to BS type applicability and then the conformance test related challenges should be discussed further. [Ericsson]

- Proposal 2: In the context of ISAC refer to: “ISAC capable BS” instead of “TRP”. [Ericsson]

- Observation 1:It is very unfortunate that the abbreviation TRP has different meanings in RAN1 and RAN4. In RAN4 TRP refer to Total Radiated Power. RAN4 should avoid referring to TRP for sensing as defined in RAN1 and instead refer to BS. [Ericsson]

- Proposal 3: Inform the other groups in RAN and SA (via LS) that in RAN4 the abbreviation TRP is already used to denote Total Radiated Power for AASs. In RAN4 the BS specification captures both radio and baseband aspects, hence TRP with “Transmission and Reception Point” is not correct in that context. [Ericsson]

- Observation 2: It is possible to start with TRP monostatic / TRP-TRP bistatic sensing and then extend to UE-TRP/TRP-UE bistatic sensing as specifications evolve. [Nokia]

- Proposal 4: RAN4 to start the sensing testability study in TRP-based sensing modes and then continue to study the testability in UE-TRP/TRP-UE bistatic sensing. [Nokia]

- Recommended WF:

  - Discuss how to handle the testing aspects for 6G sensing during SI phase

    - Postpone the conformance testing for 6G sensing [Huawei, Ericsson]

    - Continue the technical discussions in high level

  - Discuss whether to replace the TRP with ISAC capable BS in RAN4 and send LS to RAN and SA [Ericsson]

  - Discuss the priority on handling the testability for different sensing mode

    - RAN4 to start the sensing testability study in TRP-based sensing modes and then continue to study the testability in UE-TRP/TRP-UE bistatic sensing [Nokia]

    - No priority among different sensing mode

Issue 3-2: The BS type for 6G conformance testing

- Proposal 1: for TRP side testing, 1-H and 1-O TRP are suggested including both conducted and OTA testing. [CMCC]

- Proposal 2: RAN4 to consider both conductive test and radiated test for 6G sensing RF performance measurement. [Samsung]

- Proposal 3: RAN4 shall consider both conducted and OTA-based testing methodologies. Conducted testing is targeted to verify basic requirements, which can represent simplified sensing operation, while OTA-based testing is used to validate radiated performance characteristics that better reflect real-world sensing deployment scenarios.  [Vivo]

- Proposal 4: For the 6G TRP monostatic conformance testing, propose to consider both OTA and conducted testing method.  [ZTE]

- Proposal 6: RAN4 to keep both BS type 1-O and BS type 1-H as candidate BS types for 6G sensing conformance testing; at minimum, BS type 1-H to be applicable for conformance testing. [Nokia]

- Proposal 7: RAN4 to start the study of BS type 1-H for sensing RF conformance testing after the Tx/Rx switching requirements for monostatic sensing are defined. [Nokia]

- Proposal 8: to avoid reflection impact, the anechoic chamber used in OTA UE testing can be referred as baseline for the sensing mode when UE is used as receiver. [CMCC]

- Recommended WF:

  - For TRP mono-static and bi-static sensing mode, further discuss the BS type 1-H:

    - For BS type 1-H

      - Support to be studied by CMCC, [Samsung, ZTE and Nokia]

    - BS type 1-O and BS type 2-O need to be studied [agreed in WF R4#2605009]

  - For TRP-UE bi-static sensing mode, further discuss the following proposal:

    - OTA [CMCC]

  - For UE-TRP bi-static sensing mode,further discuss the following proposal:

  - For UE-UE mono-static sensing mode,further discuss the following proposal:

    - OTA [CMCC]

  - For UE-UE bi-static sensing mode, further discuss the following proposal:

    - OTA [CMCC]

Issue 3-3: The conformance testing metric for 6G sensing

- Observation 1: different from SNR/throughput testing, the reflection in testing environment will have significantly negative impact for following five sensing related requirements. [CMCC]

  - Detection Probability

  - False Alarm Probability

  - Horizontal Localization Accuracy

  - Vertical Localization Accuracy

  - Velocity Accuracy

- Proposal 1: RAN4 is suggested to study whether and how to testing above five sensing related OTA Rx requirements in the environment without any reflection, i.e. only pure LOS signal. [CMCC]

- Observation 2: For mono-static sensing, a hybrid approach using both pulsed and continuous waves is necessary. Pulsed waves enable high-power transmission for long-range detection via TDM-based SI avoidance. Complementarily, low-power continuous waves are used for short-range sensing where self-interference remains within acceptable limits. [CMCC]

- Observation 3: For bi-static sensing, continuous wave transmission is sufficient. The inherent spatial isolation between the Tx and Rx nodes allows for high-power CW transmission, achieving extensive coverage while minimizing self-interference. [CMCC]

- Proposal 2: It is proposed that RAN4 further evaluate the feasibility of utilizing only continuous waves for bi-static testing, and the feasibility of utilizing both pulsed and continuous wave support for mono-static sensing configurations. [CMCC]

- Observation 4: Different sensing modes may require different test metrics, making mode-specific evaluation essential. [Nokia]

- Proposal 3: RAN4 to identify common and mode-specific sensing test metrics depending on the considered sensing modes. [Nokia]

- Recommended WF:

  - Discuss the above proposals for the testing signal and conformance testing metric:

    - NOTE: from the feature lead perspective, this should follow the conclusion for 6G sensing requirements’ decision.

Issue 3-3: The testing framework and setup for 6G sensing

- Observation 1: A dedicated sensing testing framework is required, but its full definition depends on foundational PHY aspects such as signal structures, waveform definitions, and measurement types specified by RAN1. [Nokia]

- Proposal 1: RAN4 to study sensing testability aspects, such as the practical realization of testing framework, including definition and modeling of DUT in the different sensing modes. [Nokia]

- Proposal 2: RAN4 to study the definition and modeling of TE alongside DUT for each sensing mode as part of the sensing testing framework. [Nokia]

- Observation 2: It is possible to start with TRP monostatic / TRP-TRP bistatic sensing and then extend to UE-TRP/TRP-UE bistatic sensing as specifications evolve. [Nokia]

- Proposal 3: RAN4 shall take test complexity and implementation cost into account when finalizing the test methodology framework for 6G sensing. [Vivo]

- Proposal 4: RAN4 to adopt the test procedure shown in Figure 2.1-1 as starting point for 6G sensing conductive test. [Samsung]

Figure 2.1-1 Illustration of 6G sensing conductive test procedure

- Proposal 5: RAN4 to adopt the test procedure shown in Figure 2.1-2 as starting point for 6G sensing radiated test.[Samsung]

Figure 2.1-2 Illustration of 6G sensing radiated test procedure

- Proposal 6: For the sensing mode other than TRP monostatic conformance testing, propose to further discuss whether the assistant information from TRP or UE is needed and how the received raw signal is processed and reported. [ZTE]

- Proposal 7: RAN4 should define an appropriate test system framework to support high dynamic range and proper channel model for 6G sensing. [Vivo]

- Recommended WF:

    - For the testing framework for 6G sensing, propose to discuss the definition and functionality of TE and DUT for each sensing mode;

    - For the test setup for 6G sensing, propose to further discuss the above proposal 4 and proposal 5;

Issue 3-4: Channel modelling for 6G sensing

- Proposal 1: Propose to discuss and study how to establish the channel modelling as captured in TR 38.901 and whether any simplifications are potentially needed for the practical testing setup.  [ZTE]

- Proposal 2: RAN4 Collect vendor feedback from test system and equipment manufacturers on achievable channel model and moving target simulation capabilities. [Vivo]

- Recommended WF:

    - Propose to discuss the channel modelling for 6G sensing:

      - Conducted channel modelling if BS type 1-H for 6G sensing is agreeable

      - OTA channel modelling for FR1 and FR2-1.

Issue 3-5: Others for 6G sensing conformance testing

- Observation 1:It is essential for regulatory that characteristics, such as spurious emission, is captured properly. Therefore, conformance test should be done to reflect normal operation. [Ericsson]

- Proposal 1: For BS and UE conformance testing define test signals such that normal operation is captured for ISAC capable BS and ISAC capable UE. [Ericsson]

- Recommended WF:

    - Discuss the above proposals;

- Annex 8.10.4

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2605759 | Huawei, HiSilicon | On 6GR Sensing Coexistence |
| R4-2605799 | CATT | Discussion on Sensing coexistence studies |
| R4-2605930 | Xiaomi | View on 6G Sensing coexistence |
| R4-2605971 | vivo | Discussion on Coexistence study and simulation assumptions aspects of 6G sensing |
| R4-2606187 | Qualcomm Germany | Views on coexistence aspects for 6G sensing |
| R4-2606861 | Samsung | Discussion on coexistence and simulation assumptions of sensing for 6GR |
| R4-2606886 | ZTE Corporation, Sanechips | Discussion on coexistence study and simulation assumptions for 6G sensing |
| R4-2606908 | Ericsson | 6G ISAC on: “Coexistence study and simulation assumptions” |
| R4-2606967 | BUPT, China Telecom | Considerations on simulation assumptions for FR1 TDD sensing evaluation |
| R4-2606979 | Nokia | Coexistence aspects for sensing in 6G networks |

- Annex 8.10.5

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2605760 | Huawei, HiSilicon | On 6GR Sensing RF requirements |
| R4-2605800 | CATT | Discussion on RF requirements for 6G sensing |
| R4-2605910 | Xiaomi | Overview for 6GR ISAC spec impacts |
| R4-2605997 | vivo | Discussion on RF and RRM requirements for 6G sensing |
| R4-2606024 | CMCC | Discussion on 6G sensing requirements |
| R4-2606887 | ZTE Corporation, Sanechips | Discussion on the impacts of existing requirements and new requirements for 6G sensing |
| R4-2606909 | Ericsson | 6G ISAC on: “Impact on the existing requirements and potential new requirements (RF, RRM, etc.)” |
| R4-2607090 | Nokia | Discussion on 6G Sensing Requirements |

- Annex 8.10.6

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2605761 | Huawei, HiSilicon | On 6GR Sensing testing |
| R4-2605819 | Nokia | Testing aspects for sensing in 6G networks |
| R4-2605998 | vivo | Discussion on 6G sensing testability |
| R4-2606025 | CMCC | Discussion on 6G testing related aspects |
| R4-2606862 | Samsung | Discussion on testability of sensing for 6GR |
| R4-2606888 | ZTE Corporation, Sanechips | Discussion on testing for 6G sensing |
| R4-2606910 | Ericsson | 6G ISAC on: “Testing aspects” |