# 3GPP TSG-RAN WG4 Meeting #119	     R4-2607244

# Dalian, China, 18th – 22nd, May, 2026

Agenda item:			8.1.1

Source:	Moderator (Qualcomm)

Title:	Topic summary for [119][363] 6G_Sensing_feasiblity_general

Document for:	Information

# Introduction

The following RAN4 agenda items under the 6G SI (FS_6G_Radio) are covered in topic [363]:

- 8.10.1: General, work plan and regulatory requirement survey

- 8.10.2: Use cases and sensing modes

- 8.10.3: Feasibility study

# Topic #1: General & regulatory survey

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Observations/ Proposals |
| --- | --- | --- |
| R4-2605756 | Huawei, HiSilicon | No observations or proposals provided. |
| R4-2605931 | Xiaomi | Observation 1-1: According to 6GR RAN-P SI TR 38.914, at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs. And other sensing use cases pending on further discussion. Observation 1-2: In 6GR, target use cases, sensing modes (e.g., mono-static, bi-static) and deployment scenarios are subject to RAN/SA and RAN1's discussion and decision. Observation 1-3: With sufficient progress from other WGs, it's hard to have efficient discussion in RAN4 for ISAC Observation 2: The regulatory issues related to sensing need to be noted and continuously tracked. Proposal 1-1: RAN4 can start ISAC discussion from the typical sensing modes (e.g. TRP-TRP mono static sensing and TRP-UE bi-static sensing). |
| R4-2605970 | vivo | Proposal 1: To capture the existing regulatory information about TDD synchronization and cross-border issues in TDD operations in different regions, e.g. CEPT, as a baseline. Proposal 2: For TRP or UE mono-static mode, the regulatory survey can capture the information from TR 38.858. Proposal 3: For TRP-TRP or UE-UE bi-static mode, the dynamic TDD / asynchronized TDD regulation can be captured. Proposal 4: For TRP-UE bi-static mode, if existing reference signals in radio interface are re-used, it has no difference to current 6GR from spectrum designation. Proposal 5: To discuss if privacy/data protection regulations are part of the scope for regulatory survey or not. |
| R4-2606022 | CMCC | Observation 1: following list the sensing related regulatory state in China. |
| R4-2606858 | Samsung | Observation 1: The work plan for 6G sensing should not only consider the ending point of RAN4 6G study item, but also the industrial common understanding as well as corresponding workload. Observation 2: The current timeline of finalizing simulation assumption for all selected sensing modes might be too tight. Observation 3: RAN4 could narrow down the scope of finalizing simulation assumption discussion to TRP monostatic mode and give more time for other sensing mode simulation assumption discussions. Observation 4: Currently, ISAC is a still relatively new technology area, and there have not been any established regulations or RF requirements on this technology yet. Proposal 1: RAN4 could adopt the existing requirements in regulations as starting point or reference for 6GR ISAC discussions until there is regulation update from countries. |
| R4-2606883 | ZTE Corporation, Sanechips | Proposal 1: For the regulatory requirement, propose to consider the above CCSA regulatory information into account. Proposal 2: Propose to discuss and agree on the TR skeleton for 6G sensing in TR 38.760-4 in RAN4#119 meeting. |
| R4-2606906 | Ericsson | Proposal: It is proposed that the attached text proposal is included in TR 38.760-4. |
| R4-2606978 | Nokia | Observation 1: For US, FCC considers key technical aspects such as frequency band allocation, power limits, spectral density, and duty cycle to ensure safe and effective operation of short-range radar systems. Frequency ranges considered for radar operation are the 76-81 GHz band and the 60 GHz band. Observation 2: Regulatory aspects in Europe for the various envisaged use cases of integrated sensing and communication are being considered in EU funded projects such as Hexa-X-II and in pre-standard research such as ETSI ISG ISAC. Frequency ranges for sensing consider 0.5 GHz up to 170 GHz frequency range, both for OFDM based signals and radar. Observation 3: For China, it is expected that RF requirements will comply with regional spectrum regulations, including designated frequency usage and technical compliance. Frequency ranges considered for radar operation are the 76-79 GHz band and the 24.05-24.25 GHz band. Observation 4: IMT-2030 recommendation REC-M.2160 considers frequency bands above 92 GHz as feasible for sensing applications. Observation 5: ITU-R WP 5D is in the process of creating IMT-2030 recommendations. Following performance metrics are thereby considered for sensing: Detection probability, False alarm probability, Horizontal localization accuracy, Vertical localization accuracy, Velocity accuracy. Proposal 1: RAN4 to agree the text proposal in the Annex of R4-2606978 to be included in TR 38.760-4 for regulatory requirement survey. Proposal 2: RAN4 to take into account technical performance requirements for IMT-2030 during the feasibility study for 6G sensing performance metrics. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 6G sensing skeleton

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-1: 6G sensing TR skeleton

- Recommended WF

  - Agree on the TR skeleton for 6G sensing in TR 38.760-4 in R4-2606883.

### Sub-topic 1-2 Regulatory requirements

- TDD synchronization relevant proposals:

  - Proposal 1: To capture the existing regulatory information about TDD synchronization and cross-border issues in TDD operations in different regions, e.g. CEPT, as a baseline (Vivo).

  - Proposal 2: For TRP or UE mono-static mode, the regulatory survey can capture the information from TR 38.858 (Vivo).

- Regulatory requirements relevant proposals:

  - Proposal 1: RAN4 could adopt the existing requirements in regulations as starting point or reference for 6GR ISAC discussions until there is regulation update from countries (Samsung).

- Privacy/ data protection regulations:

  - Proposal 1: To discuss if privacy/data protection regulations are part of the scope for regulatory survey or not (Vivo).

Issue 1-2-1: Regulatory requirements

- Recommended WF

  - Follow RAN4#118bis agreement

    - To capture the regulatory requirements in 6G TR for the information only

    - Keep monitoring regulatory requirements from different regions and provide the corresponding updates in RAN4.

Issue 1-2-2: Regulatory-related info in 6G TR

- Text proposals related to regulatory survey:

  - Proposal 1: For the regulatory requirement, propose to consider the above CCSA regulatory information into account (ZTE).

  - Proposal 2: It is proposed that the attached text proposal is included in TR 38.760-4 (Ericsson).

  - Proposal 3: RAN4 to agree the text proposal in the Annex of R4-2606978 to be included in TR 38.760-4 for regulatory requirement survey (Nokia)

  - Proposal 4: RAN4 to take into account technical performance requirements for IMT-2030 during the feasibility study for 6G sensing performance metrics (Nokia)

- Recommended WF

  - Regarding regulatory section drafting, capture related regulations (if exists) for different ITU regions

    - Starting from RAN4#120, work on consolidated TPs could start

      - Consider input in R4-2606906 as a starting point

    - FFS if privacy/ data protection is in the scope of ISAC regulatory survey.

# Topic #2: Use cases and sensing modes

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2605757 | Huawei, HiSilicon | Observation 1: For the UAV sensing scenario, multiple aNB mono-static sensing achieves better performance than aNB mono-static sensing. Observation 2: For multiple aNB mono-static sensing, measurements from multiple TRPs are fused at core network, while the signal transceiving and processing procedures are identical to aNB mono-static sensing from RAN perspective. Observation 3: Multiple aNB mono-static sensing could meet the performance objectives for UAV sensing scenario in 5GA sensing study, while no additional performance objectives are identified for UAV sensing in the RAN study of 6G sensing. Proposal 1: For the study of 6G UAV sensing with multiple TRP monostatic, no impact for RAN is observed, in addition to that for TRP monostatic. Proposal 2: aNB mono-static sensing should be prioritized for the UAV sensing scenario. Proposal 3: For the study of 6G vehicle sensing, aNB-UE and UE-aNB bi-static sensing should be prioritized. Proposal 4: To focus on the study of the agreed vehicle sensing scenario. Proposal 5: The two modes are left for discussion later for RAN4 study until adequate progress is achieved in other working groups. |
| R4-2605797 | CATT | Observation 1: From RF feasibility perspective, there is no big difference between studying single monostatic TRP and multiple monostatic TRPs for UAV case. Proposal 1: multiple monostatic TRPs for UAV case should be the baseline assumption from coexistence study perspective. Proposal 2: For Vehicle (passive object) + TRP-UE bistatic, RAN4 can assume that UE is located in the fixed location initially. For example, the UE can be RSU (Road Side Unit) located in a fixed point. |
| R4-2605816 | BUPT, China Telecom | Observation 1: TRP-based ISAC has the natural advantage of coverage and networking for UAV tracing and even management, which perfectly aligns with the agreed 6G sensing target in the recent WF. Observation 2: Use case of Time sensitive service (TSC) of 6G-RAN internal could also be considered in 6G sensing study to facilitate communication. Proposal 1: In 6G, the focus should remain on TRP-based sensing modes, particularly targeting UAVs as agreed in the recent WF. Proposal 2: To overcome the physical limitations of single TRP, RAN4 should resolve the FFS and agree to include Multiple TRP monostatic and TRP-TRP bistatic modes in the feasibility study. |
| R4-2605823 | CEWiT | Proposal 1: All use cases agreed in RANP shall be considered in the RAN4 study, with initial priority given to UAV detection and tracking. Proposal 2: All six identified sensing modes shall be supported as Rel-20 study candidates in RAN4, without exclusion. If prioritisation is required, UE-involved sensing modes should be given priority due to their impact on UE RF capabilities, RRM behaviour, and testability, as well as their additional implementation challenges. |
| R4-2605964 | vivo | Observation 1: Limited by the spectrum allocation, device size and cost of UE, the self-interference problem of UE mono-static sensing is difficult to handle. Observation 2: The use case that reusing the pulse-based sensing signal for TRP mono-static sensing as reference signal for communication, poses a challenge to the receiver hardware of UE. Proposal 1: RAN4 to deprioritize the UE mono-static sensing, at least in the initial technical discussions for RAN4 6G sensing. Proposal 2: RAN4 to prioritize the indoor use case for TRP-UE bi-static sensing, and human is proposed to be prioritized as the sensing target. -FFS: single UE or multiple UEs are involved in the TRP-UE bi-static sensing. Proposal 3: For UE-TRP bi-static sensing, indoor case is prioritized, and human is proposed to be prioritized as the sensing target. Proposal 4: RAN4 to deprioritize the use case that reusing the pulse-based sensing signal for TRP mono-static sensing as reference signal for communication. |
| R4-2606026 | CMCC | Observation 1: vehicle and human rather than UAV seem more suitable as passive object for UE-TRP bistatic. |
| R4-2606185 | Qualcomm Germany | Proposal 1: RAN4 to clarify if the 1:1 association between sensing targets and sensing modes is for the purpose of defining feasibility study and coexistence evaluation scenarios. Proposal 2: RAN4 to consider AGV detection use case for UE-TRP bistatic sensing mode. Proposal 3: RAN4 to include AGV/Human detection use case via UE monostatic sensing as part of the relevant scenarios in the feasibility study of RAN4 ISAC in the 6G SI. |
| R4-2606696 | Nokia | Observation 1: Fusion of measurements from multiple TRPs was considered in the NR ISAC study as a technique to achieve the sensing performance objectives defined for the evaluation of the UAV detection use case with gNB-based monostatic sensing. Proposal 1: RAN4 to clarify the meaning of "Multiple TRP monostatic" compared to TRP-TRP multistatic sensing mode and sensing fusion considered in the NR ISAC study. Proposal 2: RAN4 to start the study of the TRP monostatic mode without considering sensing fusion aspects. These can be considered at a later stage of the study, e.g., to enhance the 6G sensing performance. Proposal 3: RAN4 to consider vehicle as the sensing target for the UE-TRP bistatic scenario in the 6G sensing feasibility and coexistence studies. |
| R4-2606859 | Samsung | Proposal 1: RAN4 to deprioritize communication assistance use case in 6G sensing until substantial progress has been made from RAN1. Proposal 2: RAN4 incorporate UE monostatic sensing mode into the 6G sensing feasibility study. |
| R4-2606884 | ZTE Corporation, Sanechips | Proposal 1: For UE-TRP bistatic case, propose to consider the vehicle (passive object) as the detection objective. Proposal 2: From the coexistence perspective, propose to focus on single TRP mono-static as baseline evaluation case. |
| R4-2606903 | Ericsson | Observation 1 The multiple monostatic TRP scenario is already allowed for sensing in 5G, according to the RAN3 discussions and agreements. Observation 2 Multiple TRPs performing monostatic sensing and hereby interfering each other is a realistic scenario. Observation 3 Using together monostatic measurements from multiple TRPs can provide non-negligeable performance gains, e.g., when a single TRP does not provide sufficient coverage within the sensing area due to antenna radiation pattern, antenna configuration, etc., which is important in the performance analysis. Proposal 1: Further clarify the earlier RAN4 agreement on the scenarios: Agreement for the scenarios (sensing modes and sensing target) for 6G sensing coexistence study from RAN4 work management perspective: - UAV (passive object), TRP monostatic, including multiple monostatic TRP - FFS on Multiple TRP monostatic. - Vehicle (passive object), TRP-UE bistatic - [TBD] (passive object), UE-TRP bistatic - UAV (passive object), TRP-TRP bistatic RAN4 doesn't preclude any 6G scenarios as agreed by RAN-P and RAN1; Proposal 2: Additional aspects, which can be considered in the scenario with multiple TRPs performing monostatic sensing measurements, can include, e.g.: - Increased interference from sensing signals; - Performance gains from combining monostatic measurements (e.g., when a single TRP does not provide sufficient coverage within the sensing area due to antenna radiation pattern, antenna configuration, etc.). Proposal 3: Further clarify the earlier RAN4 agreement on the scenarios: Agreement for the scenarios (sensing modes and sensing target) for 6G sensing coexistence/ feasibility study from RAN4 work management perspective: - UAV (passive object), TRP monostatic, including multiple monostatic TRP - Vehicle (passive object), TRP-UE bistatic (TRP transmits) - TRP Vehicle (passive object), UE-TRP bistatic (UE transmits) - UAV (passive object), TRP-TRP bistatic RAN4 doesn't preclude any 6G scenarios as agreed by RAN-P and RAN1; |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1 Use cases selection and sensing modes prioritization

- Proposal: RAN4 to deprioritize communication assistance use case in 6G sensing until substantial progress has been made from RAN1 (Samsung)

- Proposal: RAN4 incorporate UE monostatic sensing mode into the 6G sensing feasibility study (Samsung).

- Proposal: Deprioritize the use case of reusing pulse-based sensing signal for TRP monostatic as a communication reference signal, due to hardware challenges at UE receiver (Vivo).

- Proposal: RAN4 to deprioritize the UE mono-static sensing, at least in the initial technical discussions for RAN4 6G sensing (Vivo).

- Proposal: RAN4 to prioritize the indoor use case for TRP-UE bi-static sensing, and human is proposed to be prioritized as the sensing target. -FFS: single UE or multiple UEs are involved in the TRP-UE bi-static sensing (Vivo).

- Proposal: All use cases agreed in RANP shall be considered in the RAN4 study, with initial priority given to UAV detection and tracking (CEWit).

- Proposal: All six identified sensing modes shall be supported as Rel-20 study candidates in RAN4, without exclusion. If prioritisation is required, UE-involved sensing modes should be given priority due to their impact on UE RF capabilities, RRM behaviour, and testability, as well as their additional implementation challenges (CEWit).

- Proposal: aNB mono-static sensing should be prioritized for the UAV sensing scenario (Huawei).

- Proposal: For the study of 6G vehicle sensing, aNB-UE and UE-aNB bi-static sensing should be prioritized (Huawei).

- Proposal: RAN4 to clarify if the 1:1 association between sensing targets and sensing modes is for the purpose of defining feasibility study and coexistence evaluation scenarios (Qualcomm).

- Proposal: RAN4 to include AGV/Human detection use case via UE monostatic sensing as part of the relevant scenarios in the feasibility study of RAN4 ISAC in the 6G SI (Qualcomm).

- Proposal 1: In 6G, the focus should remain on TRP-based sensing modes, particularly targeting UAVs as agreed in the recent WF (BUPT/China Telecom)

- Recommended WF

  - Agree that underlying scenario/ use case for each sensing mode will be considered in the feasibility and/or coexistence studies.

  - Discussion on waveform design impacts is better addressed in feasibility and coexistence tracks separately.

  - Given no guidance from RAN on de-prioritization, all six modes could be discussed in RAN4 depending on interested companies.

    - Agreed list in RAN4#118bis on sensing targets and modes act as a starting point to have more focused discussion within RAN4.

  - No further prioritization/de-prioritization of sensing modes at this stage, unless specific infeasibility is demonstrated.

### Sub-topic 2-2 Single and multiple TRP sensing

- Proposal: For the study of 6G UAV sensing with multiple TRP monostatic, no impact for RAN is observed, in addition to that for TRP monostatic (Huawei).

- Proposal: RAN4 to clarify the meaning of "Multiple TRP monostatic" vs. TRP-TRP multistatic; start study of TRP monostatic without sensing fusion aspects (fusion can be considered later) (Nokia).

- Proposal: From coexistence perspective, focus on single TRP monostatic as the baseline evaluation case (ZTE).

- Proposal: Multiple monostatic TRPs should be the baseline assumption for the UAV coexistence study (CATT).

- Proposal: Multiple TRP monostatic should be included (remove FFS in the earlier agreement) (Ericsson).

- Proposal: For multiple TRP monostatic additional aspects, such as increased interference and performance gains from combining measurements (e.g., when a single TRP does not provide sufficient coverage within the sensing area due to antenna radiation pattern, antenna configuration, etc.), can be considered (Ericsson).

- Proposal: RAN4 should resolve the FFS and agree to include Multiple TRP monostatic and TRP-TRP bistatic modes in the feasibility study (BUPT/China Telecom).

- Recommended WF

  - Include multiple TRP monostatic under “TRP monostatic” in the feasibility and coexistence study.

    - FFS the impact of single Vs. multiple TRP monostatic considerations in RAN4 feasibility and coexistence studies, e.g., due to increased interference and performance gains from combining measurements.

### Sub-topic 2-3 Sensing targets for Bistatic modes

- Proposal: For UE-TRP bi-static sensing, indoor case is prioritized, and human is proposed to be prioritized as the sensing target (Vivo).

- Proposal: RAN4 to consider AGV detection use case for UE-TRP bistatic sensing mode (Qualcomm).

- Proposal: RAN4 to include AGV/Human detection use case via UE monostatic sensing as part of the relevant scenarios in the feasibility study of RAN4 ISAC in the 6G SI (Qualcomm).

- Proposal: For UE-TRP bistatic case, propose to consider the vehicle (passive object) as the detection objective (Nokia, ZTE, Ericsson).

- Recommended WF

  - RAN4#118bis agreement

Agreement for the scenarios (sensing modes and sensing target) for 6G sensing feasibility and coexistence study from RAN4 work management perspective: - UAV (passive object), TRP monostatic - FFS on Multiple TRP monostatic. - Vehicle (passive object), TRP-UE bistatic - [TBD] (passive object), UE-TRP bistatic - UAV (passive object), TRP-TRP bistatic RAN4 doesn’t preclude any 6G scenarios as agreed by RAN-P and RAN1;

  - Discuss following options for sensing target for UE-TRP:

    - Indoor scenarios, Human (Vivo)

    - AGV (Qualcomm)

    - Vehicle (ZTE, Ericsson, Nokia).

# Topic #3: Feasibility study

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2605758 | Huawei, HiSilicon | Observation 1: For aNB mono-static based UAV sensing scenario, aNB needs to transmit pulse and continuous wave-based signals to sense the short-ranged and long-ranged targets, respectively. Observation 2: For pulse wave-based aNB mono-static sensing, aNB transmits and receives signals in a TDM manner, and the minimum sensing range is determined by the Tx-Rx switching time interval. Observation 3: For continuous wave-based aNB mono-static sensing, aNB transmits and receives concurrently in a full-duplex manner, and needs to lower the transmit power under the restriction of Tx-Rx antenna panel isolation. Observation 4: To facilitate doppler estimation, UE needs to have the capability of phase continuity within a coherent processing interval (CPI) which crosses multiple UL and DL slots. Observation 5: For aNB-UE/UE-aNB bi-static sensing utilizing air interface synchronization, the accuracy of sensing results in terms of location or doppler estimation rely on synchronization accuracy when delay of reference path between UE and aNB is unknown a prior. Proposal 1: For aNB mono-static based UAV sensing scenario, the feasibility study should consider the following issues: - Tx-Rx switching time interval for pulse wave-based long-range sensing is restricted by the CP length of signals for continuous wave-based short-range sensing - Tx-Rx antenna panel isolation for continuous wave-based short-range sensing - Sensing signal received sensitivity for both the sensing methods. Proposal 2: For aNB-UE/UE-aNB bi-static based vehicle sensing scenario, the feasibility study should consider the following issues:  Phase continuity of UE when UL/DL switches for TDD system - Synchronization between aNB and UE - Sensing signal received sensitivity. Proposal 3: For study of 6G sensing, the prioritized bands could be restricted to 4GHz and ~7GHz for FR-1. |
| R4-2605798 | CATT | Observation 1: for UE mono-static under continuous wave scheme, 3dBm self-interference (23dBm MOP - 20dB isolation) is much higher than the -20dBm maximum input level. Observation 2: for UE mono-static under pulse wave scheme, 23dBm/26dBm MOP is too small to detect the objectives out of the PA on-off timing. Proposal 1: if RAN4 use the same 6GR UE assumptions for 6G sensing study agreed in the RAN4#118bis meeting, it's infeasible for UE mono-static under both continuous wave and pulse wave scheme. |
| R4-2605824 | Tejas Network Limited | Observation 1: In monostatic sensing, close transmitter and receiver proximity creates strong self-interference challenges. In time division operation, transmit duration, Tx to Rx switching time, receiver settling time and blanking period determine the minimum detectable range and may limit detection of nearby objects. Observation 2: Full duplex monostatic operation requires sufficient self-interference suppression through antenna isolation, analog cancellation, digital cancellation and receiver filtering. Separate Tx and Rx panels at the TRP, or separate Tx and Rx antennas at the UE, can improve isolation and reduce Tx leakage, subject to implementation constraints. Observation 3: In TRP to UE bistatic sensing, the UE receiver may need to tolerate a strong direct path component while preserving sensitivity to weak reflected echoes. In UE to TRP bistatic sensing, UE transmit timing, UE location information and UE transmit power uncertainty need consideration. Observation 4: Synchronization accuracy between TRP and UE is important for ranging and coherent sensing. Existing mechanisms may support coarse ranging, while fine sensing may require tighter time alignment, phase coherence, and calibration of RF and processing delays. Observation 5: The received reflected signal quality in bistatic sensing depends on bistatic geometry, target radar cross section, propagation loss, Doppler spread, angle of arrival, clutter and beamforming gain. Suitable channel models are needed for realistic performance evaluation. Proposal 1: The impact of transmit duration, Tx to Rx switching time, receiver settling time and blanking period on minimum detectable range and near node sensing coverage for monostatic sensing should be evaluated. Proposal 2: The total self-interference suppression budget required for full duplex monostatic operation should be analyzed, including the combined effects of separate Tx and Rx panels or antennas, antenna isolation, analog cancellation, digital cancellation and receiver filtering. Proposal 3: Receiver linearity, blocking performance, residual self-interference, ADC headroom and sensing receiver sensitivity degradation should be studied to support coexistence between sensing and communication functions. Proposal 4: RAN4 may consider quantifying receiver dynamic range requirements for bistatic sensing, taking into account direct path signal strength, weak reflected echoes, LNA compression, AGC behaviour, ADC headroom, quantization noise and blocker tolerance. Proposal 5: RAN4 may consider evaluating synchronization accuracy and reflected signal quality for bistatic sensing under realistic geometries, taking into account timing and phase alignment, RF and processing delay calibration, target radar cross section, propagation loss, Doppler spread, angle of arrival, clutter and beamforming based direct path suppression. |
| R4-2605868 | LG Electronics Inc. | Observation 1: The key principle of this contribution is that DAS beamforming should be considered not as a benchmark, but as a reference to expose RF impairments and derive algorithm-independent RF requirements in the 6G sensing feasibility study. Observation 2: While advanced algorithms can partially mask RF impairments under certain conditions, RF requirements derived based on DAS provide a conservative baseline that ensures stable sensing operation regardless of the applied algorithm. Observation 3: Element-to-element phase and gain mismatches cause coherent gain loss and sidelobe increases, directly impacting sensing characteristics, especially false alarm behavior. Observation 4: Residual SI, insufficient receiver dynamic range, and receiver non-linearity may cause weak echo masking and spurious responses, which can impact sensing performance including false alarm behavior. Observation 5: While Configured sensing bandwidth is a key factor determining resolution, its actual utilization in sensing is limited by RF characteristics. Observation 6: Advanced beamforming algorithms may improve sensing performance under certain RF characteristics; however, their performance gains are implementation-dependent and may still be limited by RF impairments. Observation 7: Existing 3GPP RF requirements are primarily designed for communication purposes and may not fully capture the specific RF impairments critical to sensing performance. - Group delay (variation): Degrading achievable range resolution within the configured sensing bandwidth. - Element-to-element phase error/noise: Impacting angle/velocity estimation and false alarms. - Residual SI: Impacting sensitivity to weak echoes. Proposal 1: It is proposed that RAN4 identifies the necessary RF parameters in the 6G sensing feasibility study using DAS beamforming as a baseline. Proposal 2: For DAS-based sensing, it is proposed to consider RF parameters such as Phase/Gain alignment, Residual SI, Dynamic range, Linearity, Phase stability, and Wideband response as key elements, and proceed with feasibility evaluation based on indicative values at the initial study assumption level. Proposal 3: Configured sensing bandwidth should be considered as supporting factor to explain deployment and capability. However, the core of this discussion is not the bandwidth itself, but the RF characteristics supporting it. Proposal 4: It is proposed that RAN4 does not directly define algorithm-dependent KPIs such as Pd/Pfa, and instead focuses the sensing feasibility discussion on RF parameters that affect the input signal quality for those KPIs. Proposal 5: It is proposed that RAN4 does not assume performance gains from advanced beamforming algorithms when identifying baseline RF parameters for 6G sensing, since such gains are implementation-dependent and may still be limited by RF impairments. Proposal 6: Acknowledging that the identified sensing-critical RF impairments are traditionally considered vendor implementation, it is proposed to discuss HOW RAN4 should approach these issues. |
| R4-2605932 | Xiaomi | Observation 1: The detailed RF impact is pending other WGs' design. Observation 2: Based on the calculation, 0.5dB sensitivity loss is acceptable for the UAV use case. Proposal 1: RAN4 can start ISAC discussion from the typical sensing modes (e.g. TRP-TRP monostatic sensing and TRP-UE bistatic sensing). Proposal 2: In initial stage, RAN4 shall focus on the study on reference architecture and RF feasibility of interference handling on receiver side from gNB side (TRP-TRP monostatic sensing) Proposal 3: RAN4 can consider "The degradation of the sensing receiver sensitivity" as the criteria. Proposal 4: RAN4 can consider the self-interference cancellation architecture/ interference cancellation methods during the feasibility study of sensing. |
| R4-2605965 | vivo | Observation 1: The self-interference problem is difficult to handle for UE mono-static sensing. Observation 2: The gap between the RSRPs of sensing signal and communication signal, raises further requirements for the sensitivity and dynamic range of the sensing receiver. Observation 3: The TO and the CFO involved in TRP-UE bi-static sensing and UE-TRP bi-static sensing can be mitigated through algorithm implementations and protocol specifications in RAN1. Proposal 1: RAN4 to deprioritize the UE mono-static sensing, at least in the initial technical discussions for RAN4 6G sensing and at least before there are much progress in RAN1. Proposal 2: RAN4 to study the requirements for the sensitivity and dynamic range of the sensing receiver, for bi-static sensing. Proposal 3: Requirements of the quality of received reflected signal for sensing, can follow that the quality of received signal for communication, at least for TRP-UE bi-static sensing and UE-TRP bi-static sensing. Proposal 4: Whether the TO and the CFO have specification impacts in RAN4, needs further study. |
| R4-2606023 | CMCC | Observation 1: higher frequency will yield reduced clutter and avoid the overlap between clutter and target echoes without degradation in sensing accuracy. Observation 2: self-interference is acceptable without blocking Rx RFFD by limiting output power, e.g. 35dBm contiguous wave for mono-static when assuming 65dB isolation between Tx and Rx for FR1. Observation 3: to maximize spectral efficiency in certain special scenario, e.g. SBFD-based ISAC scenarios, an isolation level of 80 dB for FR1 can be considered as a prerequisite to enable the simultaneous operation of sensing signal transmission and uplink communication while mitigating self-interference. Observation 4: self-interference could be distinguished in frequency domain due to doppler shift and in time domain due to shorter delay from self-interference Tx. Observation 5: self-interference is acceptable without blocking Rx RFFD without limiting output power, e.g. 58dBm contiguous wave for bi-static due to longer isolation distance. Observation 6: For bi-static sensing, By utilizing spatial filtering (beamforming), the receiver can effectively distinguish the sensing signal from the direct-path interference based on their different Angles of Arrival (AoA), eliminating the need for additional self-interference cancellation complexity. Proposal 1: band s79, around 7GHz and band s258 are suggested as example bands for sensing feasibility study. |
| R4-2606186 | Qualcomm Germany | Observation 1: RAN4 has experience on FD-like operation at the BS side given the extensive work on SBFD in NR, while for UE, more analysis might be needed for UE monostatic sensing with FD-like operation. Observation 2: To realize FD-based UE monostatic sensing, sufficient self-interference cancellation needs to be addressed via multiple cancellation stages (i.e., antenna, analog, and digital domains). RAN4 is expected to quantify potential cancellation capabilities at each stage. Observation 3: For FD-based monostatic sensing, antenna isolation alone might not be enough to prevent the saturation of the UE Rx front-end, thus, necessitating further consideration of analog SIC solutions. Observation 4: TDM-based monostatic sensing avoids self-interference and enables potential higher transmit power compared to FD-based monostatic sensing but introduces blind intervals that may impact detection of fast-moving targets. Proposal 1: RAN4 to consider both FD-based and TDM-based sensing options for monostatic sensing feasibility analysis. Proposal 2: RAN4 to consider UE monostatic sensing in the 6G study item to identify main challenges, performance impacts, and potential RF assumption enhancements from RAN4 perspective. Proposal 3: RAN4 to assess the UE monostatic feasibility considering AGV/Human detection with specific UE power class assumption and SINR operation point for reliable sensing. Proposal 4: As a starting point, RAN4 to consider typical RAN4 antenna isolation values for handheld (e.g., 15-20 dB) and CPE/ vehicular (e.g., larger than 20 dB) assuming separate Tx/Rx antennas or shared antennas for Tx/Rx with additional antenna isolation e.g., circulator. Proposal 5: RAN4 to investigate analog self-interference cancellation solutions, their corresponding SIC capabilities, and imposed cost/complexity for FD-based UE monostatic sensing Proposal 6: RAN4 to consider digital self-interference cancellation solutions as well as their corresponding SIC capabilities and additional cost for FD-based UE monostatic sensing. Proposal 7: RAN4 to discuss sensing scenario and accompanying requirements on blind range detection needed for TDM-based monostatic sensing to better quantify the needed Tx/Rx switching requirements. Proposal 8: RAN4 to identify main challenges to maintain coherency and phase continuity within the agreed CPI window. Proposal 9: RAN4 to confirm if AAS assumptions given in TR 38.922 will be the baseline for the ISAC studies within RAN4 6G SI. |
| R4-2606268 | BUPT, China Telecom | Observation 1: Collaborative Multi-TRP sensing can be realized by introducing a Sensing Function (SF) in the Core Network or an Edge Sensing Function (ESF) in the RAN. Proposal 1: Based on the significant performance gains observed in both ranging and velocity RMSE, RAN4 should confirm the study of Multiple TRP monostatic (resolving the FFS from #118bis) to advance beyond the 5G-A focus on single base station sensing. |
| R4-2606372 | China Unicom | Proposal: to include multiple TRP monostatic sensing into the current study scope of 6G sensing feasibility study. |
| R4-2606860 | Samsung | Observation 1: The common understanding is that the sensing task will reuse the communication hardware including antennas, which means that there would be no additional antenna array to be installed for sensing task. Observation 2: For UE monostatic sensing, since the antenna elements for UE is much less than that of BS, there would be stringent trade-off between near-target sensing capabilities and angular resolution. Observation 3: The self-interference encountered in monostatic sensing would only be the interference signals from sensing Tx antennas to sensing Rx antennas for separate antenna configuration scenarios. Observation 4: When TRP is using separate antenna configuration, the received echo signals at the TRP fall into the exact same frequency band as transmitted sensing signals, and thus are subject to co-channel co-band self-interference from the TRP transmitter. Observation 5: For self-interference cancellation in monostatic sensing, the frequency isolation schemes are incapable of cancel sensing interference signal, only spatial isolation and digital cancellation schemes can be used. Observation 6: The required RSIC budget for the communication system is too stringent and not suitable for sensing task. Observation 7: For velocity sensing in urban macro scenario using 7GHz devices, the minimal observation window is calculated to be 5.36ms and the maximum frequency offset is at least ±13.33ppb within the observation window. Observation 8: For velocity sensing in indoor scenario using 7GHz devices, the minimal observation window is calculated to be 10.72ms and the maximum frequency offset is at least ±6.67ppb within the observation window. Observation 9: For the sensing tasks conducted across different devices, the frequency accuracy requirement could be even harder to be guaranteed. Proposal 1: RAN4 to consider both separate and integrated antenna panel configuration in 6G sensing feasibility study. Proposal 2: RAN4 to consider separate antenna panel (Option 1) as baseline configuration for TRP monostatic sensing in 6G study. Proposal 3: RAN4 to priority TDM mode for communication and sensing task multiplexing within the 6G sensing feasibility and coexistence study. Proposal 4: RAN4 to use isolation values in Rel-18 SBFD study as starting point for 6G sensing self-interference discussion. Proposal 5: RAN4 to design a new required RSIC budget dedicated for sensing task. Proposal 6: RAN4 to investigate the frequency accuracy requirement for speed/doppler estimation for monostatic sensing in feasibility study. Proposal 7: RAN4 to investigate the minimal clock synchronization requirements for different bistatic sensing scenarios. Proposal 8: RAN4 should investigate frequency accuracy requirement for speed/doppler estimation for bistatic sensing in feasibility study. |
| R4-2606885 | ZTE Corporation, Sanechips | Proposal 1: For the feasibility study for 6G sensing, propose to discuss this issue according to different sense mode as shown the above table. Sensing mode; RF feasibility. TRP mono-static; - Solution 1: TDM method. For this solution, there are no much feasibility issues since Tx and Rx are not working simultaneously, TDD ON-OFF transition period might need to be revisited since large switching time will cause lager blind detection areas in the airspace close to the sensing TRP. - Solution 2: Full duplexer method. For this solution, then TRP should support the full duplex mode to have Tx and Rx operating simultaneously. Similar as SBFD operation, the RF feasibility should be provided.. TRP bi-static; - TRP Tx and Rx is not working simultaneously, therefore there are no much feasibility problems, however from the timing sync performance from two TRPs should be further discussed since the current timing error among different TRPs are 3us which is way too relaxed for sensing operation.. TRP-UE bi-static; - Whether the location information of TRP could be accessible to UE side; - Whether the DL timing estimation error from UE has big impacts on the measurement accuracy. The existing UL timing error requirement is partially decided by DL SSB timing estimation error.. UE-TRP bi-static; - Whether the location information of UE could be accessible to UE side due to the personal information privacy; - Whether the UL timing estimation error from UE has big impacts on the measurement accuracy. The existing UL timing error requirement is partially decided by DL SSB timing estimation error. - NOTE: this case is similar as Rel-17 SRS transmission based positioning, not sure any major difference.. UE monostatic; - Similar as TRP mono-static. UE bi-static; - Similar as TRP bi-static. Proposal 2: For the TRP mono-static with solution 1 TDM operation, consider the RF feasibility evaluation from the receiver’s in-channel linearity and also out of carrier blocking performance. Proposal 3: For the TRP mono-static with solution 2 full duplex operation, consider the following example table to check its RF feasibility including fully overlapping transmission/reception between DL and UL. FR1; ZTE. BS class; Wide Area BS Full duplex mode. BS TX Power dBPTX = ① dBm; 38 dBm. Component capability and parameters; Frequency isolation at TX; Frequency isolation capability dBαSI-frequency, TX = ② dBc; N/A. Frequency isolation techniques used; N/A. Spatial isolation; Spatial isolation capability dBαSI-spatial = ③ dBc; 65 dBc. Spatial isolation techniques used; TX/RX panel separation and RF barrier structure. TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc; TBD. DL EIRP impact due to beam nulling in TX sub-band; Limited, ~0dB. Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1); Note: provided by ①-②-③-⑨ dBm. RF IC and other tech. (before LNA); RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc; N/A. RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc; N/A. RF IC techniques and other tech. (before LNA); N/A. Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA; N/A. Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1); TBD Note: provided by ①-③-④-⑤dBm. Blocker Suppression at RX; Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc; N/A. Frequency isolation techniques; N/A. RX IMD; Rx IIP3 capability (dBm); TBD. Rx IM3 contribution (dBm). Other RX; Any other RX impacts if significant (e.g. ADC noise, phase noise etc.); N/A. Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2); TBD Note: provided by ①-③-④-⑤-⑥dBm. RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc; TBD. RX sensitivity degradation caused by RX beam nulling; TBD. Digital IC dBαSI-digtial = ⑦ dBc; TBD. Overall RSIC capability dBRSIC (Note 1); TBD. Noise floor ⑩dBm; TBD/100MHz. Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB); TBD. Required RSIC budget (①-⑪dBc); TBD. Guardband assumption (if exist); N/A. bandwidth over which suppression is achieved; 100MHz. Others. Proposal 4: For the TRP bi-static sensing mode, propose to discuss the impacts of potential timing synchronization performance on the sensing results and also the impacts of 6G sensing signal on blocking performance within the carrier. Proposal 5: For TRP-UE bi-static sensing mode, propose to discuss the per UE DL timing performance and DL timing difference among different UEs for the same target object measurement and its impacts on the sensing results. Proposal 6: For TRP-UE bi-static sensing mode, propose to discuss UE’s receiver dynamic range issue until there are sufficient evaluation results from 6G sensing SLS simulation evaluation. |
| R4-2606907 | Ericsson | Observation 1: Bi-static sensing relies on relative synchronization between local clusters of neighboring transmitting and receiving nodes, hence accurate absolute time, based on a global common time reference such as GNSS, is not necessary for functions like ranging and positioning of an object. Observation 2: The crucial aspect of bistatic/multistatic sensing is that it achieves its key performance requirements, rather than focusing on the underlying synchronization method itself. Various alternatives and implementations may be possible, so the standard should permit flexibility in implementation. Observation 3: The relationship between bistatic sensing performance and synchronization methods and its performance level is complex and highly context-dependent. No single synchronization requirement can adequately represent all use cases and their varying performance levels, a value based on the most challenging dimensioning can lead to increased costs and complexity, and which will not be necessary in most scenarios. Observation 4: Most sensing applications do not require strict node-to-node synchronization. Instead, by determining the precise relationship between nodes and compensating for any synchronization impairments at the application level, one can achieve better performance and simplify implementation. This approach reduces complexity by eliminating error sources associated with managing relative synchronization and the need for considering synchronization hierarchies. Observation 5: BS monostatic sensing covers diverse use cases and implementation methods, resulting in a wide range of sensing performance requirements. There is no single switching time threshold that can universally represent all scenarios, as constraints such as TRX switching time in monostatic radar systems depend on specific use cases, actual implementations, and possible integration with other nodes. Therefore, it is essential to prioritize achieving the required sensing performance rather than adhering to a fixed switching time figure. Proposal 1: For BS-to-BS bistatic sensing identify relevant design aspects that can be of relevance for feasibility study. Proposal 2: There is no synchronization issue for intra-BS TRP-TRP bistatic/multi-static sensing with respect to existing synchronization possibilities. Proposal 3: For inter-BS TRP-TRP bistatic/multistatic sensing the synchronization solutions should be built considering the following factors: - Accurate absolute time, based on a global common time reference such as GNSS, is not necessary for functions like detection, ranging, and positioning of an object. - The standard should not restrict implementation to any specific synchronization method. - The synchronization needs are different for different sensing tasks and scenarios; the assumption on the need for tight synchronization in general for sensing will lead to unnecessary increased costs and complexity in most cases. - By determining the precise relationship between nodes and compensating for any synchronization impairments at the application level one can achieve better performance and simplify implementation. Proposal 4: TRP-TRP bistatic/multistatic sensing is feasible from the synchronization point of view, and a variety of solution approaches exist. |
| R4-2606992 | OPPO | Proposal 1: The following frequency bands can be considered as 6G sensing feasibility study as the starting point. - 4GHz: n79 - 7GHz: a 400MHz frequency range for initial study, e.g. 7GHz-7.4GHz - 30GHz: selected from n257, n258 and n261 Proposal 2: Consider -85.5dBm ~ -20dBm as the receiving dynamic range when UE serves as the receiving node in TRP-UE bi-static mode. |
| R4-2607089 | Nokia | Observation 1: Conventional shared-antenna Tx/Rx architectures following legacy TDD switching are not suitable for monostatic sensing, as Tx/Rx switching delays lead to prohibitively large sensing blind ranges. Proposal 1: RAN4 to study tighter Tx/Rx switching time values for TRP mono-static sensing mode. Proposal 2: RAN4 to study Tx/Rx architectures, such as separate Tx and Rx antennas, to mitigate switching limitations and enable practical TRP monostatic sensing performance. Proposal 3: RAN4 to conduct studies on the synchronization accuracy between TRPs and between UE and TRP for enabling bi-static sensing, taking into account that conventional over-the-air network-UE synchronization mechanisms may be insufficient and that alternative synchronization approaches may need to be considered. Proposal 4: RAN4 to consider using GPS/GNSS reference clock time stamps at the transmitter and receiver to improve sensing accuracy. |

## Open issues summary

### Sub-topic 3-2 Frequency bands for sensing feasibility study

- Proposal: For study of 6G sensing, the prioritized bands could be restricted to 4GHz and ~7GHz for FR-1 (Huawei).

- Proposal: band s79, around 7GHz and band s258 are suggested as example bands for sensing feasibility study (CMCC).

- Proposal: The following frequency bands can be considered as 6G sensing feasibility study as the starting point (Oppo)

  - - 4GHz: n79

  - ~7GHz: a 400MHz frequency range for initial study, e.g. 7GHz-7.4GHz

  - 30GHz: selected from n257, n258 and n261

- Recommended WF

  - To manage workload, proposal to focus on an example band for FR1 and FR2-1.

  - Proposed example bands:

    - FR1: 7GHz  (selected as the FR1 example band given it is a new 6G candidate band)

    - FR2: 30 GHz

  - Companies are encouraged to provide feasibility study for other bands in case feasibility outcome will be different compared to the example bands.

### Sub-topic 3-1 Feasibility evaluation framework

- Proposal: Receiver linearity, blocking performance, residual self-interference, ADC headroom and sensing receiver sensitivity degradation should be studied to support coexistence between sensing and communication functions (Tejas).

- Proposal: It is proposed that RAN4 identifies the necessary RF parameters in the 6G sensing feasibility study using DAS beamforming as a baseline (LGE).

- Proposal: It is proposed that RAN4 does not directly define algorithm-dependent KPIs such as Pd/Pfa, and instead focuses the sensing feasibility discussion on RF parameters that affect the input signal quality for those KPIs (LGE).

- Proposal: It is proposed that RAN4 does not assume performance gains from advanced beamforming algorithms when identifying baseline RF parameters for 6G sensing, since such gains are implementation-dependent and may still be limited by RF impairments (LGE).

- Proposal: It is proposed to consider RF parameters such as Phase/Gain alignment, Residual SI, Dynamic range, Linearity, Phase stability, and Wideband response as key elements, and proceed with feasibility evaluation based on indicative values at the initial study assumption level (LGE).

- Proposal: Configured sensing bandwidth should be considered as supporting factor to explain deployment and capability. However, the core of this discussion is not the bandwidth itself, but the RF characteristics supporting it (LGE).

- Proposal: Acknowledging that the identified sensing-critical RF impairments are traditionally considered vendor implementation, it is proposed to discuss HOW RAN4 should approach these issues (LGE).

- Proposal: RAN4 can start ISAC discussion from the typical sensing modes (e.g. TRP-TRP monostatic sensing and TRP-UE bistatic sensing) (Xiaomi).

- Proposal: RAN4 to consider both FD-based and TDM-based sensing options for monostatic sensing feasibility analysis (Qualcomm).

- Proposal: RAN4 to confirm if AAS assumptions given in TR 38.922 will be the baseline for the ISAC studies within RAN4 6G SI (Qualcomm)

- Proposal: RAN4 to consider both separate and integrated antenna panel configuration in 6G sensing feasibility study (Samsung).

- Proposal: RAN4 to priority TDM mode for communication and sensing task multiplexing within the 6G sensing feasibility and coexistence study (Samsung).

- Proposal : For the feasibility study for 6G sensing, propose to discuss this issue according to different sense mode as shown the above table (ZTE).

| Sensing mode | RF feasibility |
| --- | --- |
| TRP mono-static | - Solution 1: TDM method. For this solution, there are no much feasibility issues since Tx and Rx are not working simultaneously, TDD ON-OFF transition period might need to be revisited since large switching time will cause lager blind detection areas in the airspace close to the sensing TRP. - Solution 2: Full duplexer method. For this solution, then TRP should support the full duplex mode to have Tx and Rx operating simultaneously. Similar as SBFD operation, the RF feasibility should be provided. |
| TRP bi-static | - TRP Tx and Rx is not working simultaneously, therefore there are no much feasibility problems, however from the timing sync performance from two TRPs should be further discussed since the current timing error among different TRPs are 3us which is way too relaxed for sensing operation. |
| TRP-UE bi-static | - Whether the location information of TRP could be accessible to UE side; - Whether the DL timing estimation error from UE has big impacts on the measurement accuracy. The existing UL timing error requirement is partially decided by DL SSB timing estimation error. |
| UE-TRP bi-static | - Whether the location information of UE could be accessible to UE side due to the personal information privacy; - Whether the UL timing estimation error from UE has big impacts on the measurement accuracy. The existing UL timing error requirement is partially decided by DL SSB timing estimation error. - NOTE: this case is similar as Rel-17 SRS transmission based positioning, not sure any major difference. |
| UE monostatic | - Similar as TRP mono-static |
| UE bi-static | - Similar as TRP bi-static |

- Proposal: For BS-to-BS bistatic sensing identify relevant design aspects that can be of relevance for feasibility study (Ericsson)

- Recommended WF

  - Companies are encouraged to provide feasibility analysis including:

    - Sensing mode and scenario

    - Criteria for feasibility determination (e.g., sensing receiver sensitivity degradation, sensing performance KPI degradation)

    - System parameters and reference architecture

    - RF/RRM impacts if applicable

    - Other aspects are not precluded at this stage

  - Given ongoing discussion in RAN1 on waveform and integration, both FD-based and TDM-based solutions could be investigated in the RAN4 feasibility study.

### Sub-topic 3-3 Monostatic sensing (TRP & UE) feasibility aspects

#### aNB monostatic sensing

- Proposal: For aNB mono-static based UAV sensing scenario, the feasibility study should consider the following issues (Huawei)

  - Tx-Rx switching time interval for pulse wave-based long-range sensing is restricted by the CP length of signals for continuous wave-based short-range sensing

  - Tx-Rx antenna panel isolation for continuous wave-based short-range sensing

  - Sensing signal received sensitivity for both the sensing methods.

- Proposal: The impact of transmit duration, Tx to Rx switching time, receiver settling time and blanking period on minimum detectable range and near node sensing coverage for monostatic sensing should be evaluated (Tejas)

- Proposal: The total self-interference suppression budget required for full duplex monostatic operation should be analyzed, including the combined effects of separate Tx and Rx panels or antennas, antenna isolation, analog cancellation, digital cancellation and receiver filtering (Tejas).

- Proposal: In initial stage, RAN4 shall focus on the study on reference architecture and RF feasibility of interference handling on receiver side from gNB side (TRP-TRP monostatic sensing) (Xiaomi).

- Proposal: RAN4 can consider the self-interference cancellation architecture/ interference cancellation methods during the feasibility study of sensing (Xiaomi).

- Proposal: RAN4 to consider separate antenna panel (Option 1) as baseline configuration for TRP monostatic sensing in 6G study (Samsung).

- Proposal: RAN4 to use isolation values in Rel-18 SBFD study as starting point for 6G sensing self-interference discussion (Samsung).

- Proposal: RAN4 to design a new required RSIC budget dedicated for sensing task (Samsung).

- Proposal: RAN4 to investigate the frequency accuracy requirement for speed/doppler estimation for monostatic sensing in feasibility study (Samsung).

- Proposal: For the TRP mono-static with solution 1 TDM operation, consider the RF feasibility evaluation from the receiver’s in-channel linearity and also out of carrier blocking performance (ZTE).

- Proposal: For the TRP mono-static with solution 2 full duplex operation, consider the following example table to check its RF feasibility including fully overlapping transmission/reception between DL and UL (ZTE)

| FR1 |  |  |  | ZTE |  |
| --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS Full duplex mode |  |
| BS TX Power dBPTX = ① dBm |  |  |  | 38 dBm |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | N/A |  |
|  |  | Frequency isolation techniques used |  | N/A |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 65 dBc |  |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | TBD |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | Note: provided by ①-②-③-⑨ dBm |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | N/A |  |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | N/A |  |
|  |  | RF IC techniques and other tech. (before LNA) |  | N/A |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | N/A |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | TBD Note: provided by ①-③-④-⑤dBm |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | N/A |  |
|  |  | Frequency isolation techniques |  | N/A |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | TBD |  |
|  |  |  | Rx IM3 contribution (dBm) |  |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | TBD Note: provided by ①-③-④-⑤-⑥dBm |  |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | TBD |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | TBD |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | TBD |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | TBD |  |
| Noise floor ⑩dBm |  |  |  | TBD/100MHz |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | TBD |  |
| Required RSIC budget (①-⑪dBc) |  |  |  | TBD |  |
| Guardband assumption (if exist) |  |  |  | N/A |  |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |  |
| Others |  |  |  |  |  |

- Proposal: RAN4 to study tighter Tx/Rx switching time values for TRP mono-static sensing mode (Nokia).

- Proposal: RAN4 to study Tx/Rx architectures, such as separate Tx and Rx antennas, to mitigate switching limitations and enable practical TRP monostatic sensing performance (Nokia).

#### UE monostatic sensing

- Proposal: if RAN4 use the same 6GR UE assumptions for 6G sensing study agreed in the RAN4#118bis meeting, it's infeasible for UE mono-static under both continuous wave and pulse wave scheme (CATT).

- Proposal: RAN4 to deprioritize the UE mono-static sensing, at least in the initial technical discussions for RAN4 6G sensing and at least before there are much progress in RAN1 (Vivo).

- Proposal: RAN4 to consider UE monostatic sensing in the 6G study item to identify main challenges, performance impacts, and potential RF assumption enhancements from RAN4 perspective (Qualcomm).

- Proposal: RAN4 to assess the UE monostatic feasibility considering AGV/Human detection with specific UE power class assumption and SINR operation point for reliable sensing (Qualcomm).

- Proposal: RAN4 to investigate analog self-interference cancellation solutions, their corresponding SIC capabilities, and imposed cost/complexity for FD-based UE monostatic sensing (Qualcomm).

- Proposal: RAN4 to consider digital self-interference cancellation solutions as well as their corresponding SIC capabilities and additional cost for FD-based UE monostatic sensing (Qualcomm).

- Proposal: RAN4 to discuss sensing scenario and accompanying requirements on blind range detection needed for TDM-based monostatic sensing to better quantify the needed Tx/Rx switching requirements (Qualcomm).

- Proposal: RAN4 to identify main challenges to maintain coherency and phase continuity within the agreed CPI window (Qualcomm).

- Recommended WF

  - For monostatic (aNB-based and UE-based) sensing, the following aspects to be further studied within RAN4:

    - For Full duplex-based design

      - Tx-Rx panel/ element isolation and reference Tx/Rx antenna architecture

      - Total achieved self-interference cancellation and reference RF architecture

      - Residual self-interference cancellation budget calculation for sensing (starting point is the self-interference framework for Rel-18 SBFD-capable BS in TR 38.858).

      - Frequency accuracy requirement for speed/doppler estimation

      - Other aspects not precluded

    - For time division-based design

      - Tx-Rx switching time, Tx duration, and blanking period impact on detectable range performance

      - Sensing signal received sensitivity

      - Rx in-channel linearity and out of carrier blocking performance

      - Frequency accuracy requirement for speed/doppler estimation

      - Other aspects not precluded

  - For UE monostatic sensing, further discussion on challenges and potential enhancements to be investigated.

### Sub-topic 3-4 Bistatic sensing feasibility aspects

- Proposal: For aNB-UE/UE-aNB bi-static based vehicle sensing scenario, the feasibility study should consider the following issues (Huawei)

  - Phase continuity of UE when UL/DL switches for TDD system

  - Synchronization between aNB and UE

  - Sensing signal received sensitivity.

- Proposal: RAN4 may consider quantifying receiver dynamic range requirements for bistatic sensing, taking into account direct path signal strength, weak reflected echoes, LNA compression, AGC behaviour, ADC headroom, quantization noise and blocker tolerance (Tejas).

- Proposal: RAN4 may consider evaluating synchronization accuracy and reflected signal quality for bistatic sensing under realistic geometries, taking into account timing and phase alignment, RF and processing delay calibration, target radar cross section, propagation loss, Doppler spread, angle of arrival, clutter and beamforming based direct path suppression (Tejas).

- Proposal: RAN4 to study the requirements for the sensitivity and dynamic range of the sensing receiver, for bi-static sensing (Vivo).

- Proposal: Requirements of the quality of received reflected signal for sensing, can follow that the quality of received signal for communication, at least for TRP-UE bi-static sensing and UE-TRP bi-static sensing (Vivo).

- Proposal: Whether the TO and the CFO have specification impacts in RAN4, needs further study (Vivo).

- Proposal: to include multiple TRP monostatic sensing into the current study scope of 6G sensing feasibility study (China Unicom).

- Proposal: RAN4 to investigate the minimal clock synchronization requirements for different bistatic sensing scenarios (Samsung).

- Proposal: RAN4 should investigate frequency accuracy requirement for speed/doppler estimation for bistatic sensing in feasibility study (Samsung).

- Proposal: For the TRP bi-static sensing mode, propose to discuss the impacts of potential timing synchronization performance on the sensing results and also the impacts of 6G sensing signal on blocking performance within the carrier (ZTE).

- Proposal: For TRP-UE bi-static sensing mode, propose to discuss the per UE DL timing performance and DL timing difference among different UEs for the same target object measurement and its impacts on the sensing results (ZTE).

- Proposal: For TRP-UE bi-static sensing mode, propose to discuss UE’s receiver dynamic range issue until there are sufficient evaluation results from 6G sensing SLS simulation evaluation (ZTE).

- Proposal: For BS-to-BS bistatic sensing identify relevant design aspects that can be of relevance for feasibility study (Ericsson).

- Proposal: There is no synchronization issue for intra-BS TRP-TRP bistatic/multi-static sensing with respect to existing synchronization possibilities (Ericsson).

- Proposal: For inter-BS TRP-TRP bistatic/multistatic sensing the synchronization solutions should be built considering the following factors (Ericsson):

  - Accurate absolute time, based on a global common time reference such as GNSS, is not necessary for functions like detection, ranging, and positioning of an object.

  - The standard should not restrict implementation to any specific synchronization method.

  - The synchronization needs are different for different sensing tasks and scenarios; the assumption on the need for tight synchronization in general for sensing will lead to unnecessary increased costs and complexity in most cases.

By determining the precise relationship between nodes and compensating for any synchronization impairments at the application level one can achieve better performance and simplify implementation.

- Proposal: TRP-TRP bistatic/multistatic sensing is feasible from the synchronization point of view, and a variety of solution approaches exist (Ericsson).

- Proposal: Consider -85.5dBm ~ -20dBm as the receiving dynamic range when UE serves as the receiving node in TRP-UE bi-static mode (Oppo).

- Proposal: RAN4 to conduct studies on the synchronization accuracy between TRPs and between UE and TRP for enabling bi-static sensing, taking into account that conventional over-the-air network-UE synchronization mechanisms may be insufficient and that alternative synchronization approaches may need to be considered (Nokia).

- Proposal: RAN4 to consider using GPS/GNSS reference clock time stamps at the transmitter and receiver to improve sensing accuracy (Nokia).

- Recommended WF

  - Aspects to be further studies for bistatic sensing (companies to clarify specific bistatic sensing mode investigated)

    - Timing synchronization between Tx and Rx

    - Phase continuity

    - Rx sensitivity and dynamic range

    - Reflected signal quality

    - Frequency accuracy for speed/Doppler estimation

    - Impact of 6G sensing on In-carrier blocking performance

    - Timing offset and carrier frequency offset

    - Location information accessibility and implications on privacy

    - Impact of DL/ UL timing estimation error on measurement accuracy

    - Other aspects are not precluded