3GPP TSG-RAN WG4 Meeting#119                R4-2607742

Dalian, CN, May 18 - 22, 2026

Agenda item:			8.1.1

Source:	ZTE Corporation

Title:	Way Forward for [119][362] 6G_Sensing_coex_test_req

Document for:	Approval

WF

Co-existence simulation scenario

Agreement for deployment scenario of the TRP/BS mono-static, bi-static sensing mode:

- Urban Macro

- FFS for SMa and rural area

Agreement for candidate frequency range of the TRP/BS mono-static, bi-static sensing mode:

- 4GHz

- 7GHz

- 30GHz

Agreement for sensing signal bandwidth of the TRP/BS mono-static, bi-static sensing mode:

- 100MHz for 4GHz

  - FFS for 200MHz and [400MHz(optional)] and depends on the outcome of system parameter discussion.

- 100MHz, 200MHz and 400MHz(optional) for 7GHz

- 200MHz, 400MHz for 30GHz

Agreement for TRP bi-static sensing mode:

- For TN network, reuse the same assumption as TRP/BS mono-static sensing mode;

- For ISAC BS network, reuse the same assumption of antenna configuration of TDM as defined for TRP/BS mono-static sensing mode;

Agreement for deployment scenario of the TRP-UE,bi-static sensing mode/Vehicle as passive objective:

- FFS which deployment scenario to consider between Urban Macro highway and Urban Macro Manhattan grid

- FFS for SMa and rural area

Agreement for TRP-UE bistatic sensing mode:

- For TN network, reuse the same assumption as TRP/BS mono-static sensing mode;

- For ISAC BS network, FFS

FFS for the indoor scenario

Agreement for Network layout model

5.2A.1.1	Urban macro and FFS for Sub-urban Macro in FR1

Details on network layout model are listed in Table 5.2A.1.1-1 and 5.2A.1.1-2.

Table 5.2A.1.1-1: Single operator layout for urban macro for TN network at 4GHz and 7GHz [TR 38.921]

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.5 km (urban) (Note 1) FFS for 0.9 km (suburban) FFS for Rural area | NOTE: 0.5km is to aligned with the simulation assumption for 7GHz as captured in WF R4-2605008 |
| BS antenna height |  | 20 m (urban) 25 m (suburban) |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa, FFS for Sma | As defined in TR 38.901 |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Table 5.2.1.1-1: Single operator layout for urban macro for ISAC network at 4GHz and 7GHz of 6G sensing BS

| Parameters | Values | Remark |
| --- | --- | --- |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance | 0.5 km (urban) (Note 1) FFS for 0.9 km (suburban) FFS for Rural area |  |
| BS antenna height | 20 m (urban) 25 m (suburban) |  |
| Beam steering | Boresight direction for Tx/Rx calibration |  |
| Channel model | UMa-AV , FFS for Sma | As defined in TR 38.901 |
| Shadowing correlation | Between cells: 1.0 Between sites: 1.0 For the calibration |  |

Additional simulation parameters shown below. Reference (RAN1 ISAC TR 38.765 Annex A).

| Sensing target | Target type | UAV with small size (0.3m x 0.4m x 0.2m) |
| --- | --- | --- |
|  | 3D distribution | N targets per sector per sector - N = 1 for the pathloss calibration purpose Horizontal plane: Uniformly distributed in a sector Vertical plane: Uniformly distributed between 25m and 300m for the calibration |
|  | Mobility | Horizontal speed: uniformly distributed between 0 and 180km/h Vertical speed: 0km/h NOTE: for the RAN4 calibration, the mobility assumption is not needed. |
|  | Minimum BS-target 3D distance | 10 m |
|  | Minimum target-target (3D) distance | 10 m |
|  | Outdoor/indoor proportion | 100% outdoor |
|  | LOS/NLOS | LOS and NLOS (NLOS based on LoS probability calculation in TR 38.901). |
|  | Orientation | Random in horizontal domain |
|  | RCS model | RCS model 1 for UAV with small size |
| NOTE 1: The above options are calculated with BS_maxpower = BS Rx saturation power + antenna isolation by assuming the BS Rx saturation power = -28dBm and the antenna isolation = 65dB and 80dB, respectively. |  |  |

5.2A.1.2	Urban macro in FR2-1

Details on network layout model are listed in Table 5.2A.1.2-1 and 5.2A.1.2-2.

Table 5.2A.1.2-1: Single operator layout for urban macro for TN UE at 30GHz

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.2 km (urban) (Note 1) |  |
| BS antenna height |  | 25 m (urban) |  |
| UE location | Outdoor/indoor | Outdoor |  |
|  | Indoor UE ratio | 0% |  |
|  | Low/high Penetration loss ratio | N/A |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa | As defined in TR 38.901 |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Table 5.2.1.1-1: Single operator layout for urban macro for ISAC network at 30GHz

| Parameters | Values | Remark |
| --- | --- | --- |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance | 0.2 km (urban) (Note 1) FFS for 0.9 km (suburban) FFS for Rural area |  |
| BS antenna height | 25 m (urban) |  |
| Beam steering | Boresight direction for Tx/Rx calibration |  |
| Channel model | UMa-AV ,FFS for Sma | As defined in TR 38.901 |
| Shadowing correlation | Between cells: 1.0 Between sites: 1.0 For the calibration | Further discuss the correlation level |

Propagation model

Agreement:

Reuse pathloss model, LoS model, and penetration losses in Section 7.4 in TR 38.901

5.2A.2.4	Path loss for UAV

| Parameters | Values |
| --- | --- |
| Coupling loss for target channel | power scaling factor (pathloss, shadow fading, and RCS component σM included): LTX−SPST−RX=PLdBd1+PLdBd2+10lgc24πf2−10lgσRCS,A+SFdB,1+SFdB,2 For PLdB(d1) and PLdB(d2), follow the pathloss model in the above clause; For SFdB,1 and SFdB,2 follow the SF model in the above clause; |
| STX/SRX selection | Best N = 1 STX-SRX pairs to be selected for the target. NOTE: [minimum coupling loss for the UAV and TRP and further discuss with RAN1 how these parameters are used in the simulation] see note 1 |
| Metrics | Coupling loss for target channel |
| NOTE 1: Based on the STX-SRX pairs with the smallest power scaling factor of the target channel. |  |

  - Where σRCS,A (σRCS=σMσDσS). is captured in TR 38.901 as following:

| Sensing target | 10lgσM (dBsm) | σσS_dB (dB) |
| --- | --- | --- |
| UAV with small size | -12.81 | 3.74 |

  - σD=1 for the calibration

Antenna configuration for NR BS

Agreement:

5.2A.3.1	Antenna configuration for NR BS

For 4GHz, align with antenna array parameters of legacy NR BS as defined in clause 4.4.1.2 in TR 38.922

For 7GHz, with 7GHz coexistence simulation assumption as agreed in RAN4#119, FFS for SMa

For 30GHz,

Antenna array parameters for 30GHz from SBFD TR 38.858

| Parameter | Urban Macro |  |  |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 8.0 |  |  |
| Horizontal/vertical 3 dB beam width of single element (degree) | 65º for H 65º for V |  |  |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |  |  |
| Antenna polarization | Linear ±45º |  |  |
| Antenna array configuration (Row × Column) | 8 × 16 elements |  |  |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.5 of wavelength for V |  |  |
| Number of element rows in sub-array | 1 |  |  |
| Vertical element separation in sub-array (dv,sub) | 0.5 of wavelength of V |  |  |
| Total TRP (dBm) (Note 3) | 30dBm |  |  |
| Mechanical down-tilt (degrees) | 3 |  |  |

5.2A.3.2	Antenna configuration for NR UE

For FR1 UE at 4GHz and 7GHz, use the omni antenna assumption as baseline assumption and further discuss whether directivity antenna could be used for FR1 UE.

For FR2-1 UE, propose to consider the following assumption:

FR2-1 UE antenna element pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

Other simulation assumptions for FR1 and FR2-1 UE.

| Parameter | FR1 | FR2-1 |
| --- | --- | --- |
| Maximum transmitter power | Calibration to assume PC2 (26 dBm) | 22.4 dBm (EIRP) |
| Minimum transmitter power | -33 dBm (TS 38.101-1, Table 6.3.1-1) | -40 dBm |

Antenna configuration for ISAC BS

Agreement:

For full-duplex operation mode with different Tx/Rx panels:

Antenna array parameters of IASC BS for 4GHz

| Parameter | Macro urban |  |  |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |  |  |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |  |  |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |  |  |
| Antenna polarization | Linear ±45º |  |  |
| Antenna sub-array configuration (Row × Column) | Tx panel: 2 × 8 elements Rx panel 2 × 8 elements |  |  |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V |  |  |
| Number of element rows in sub-array | 3 |  |  |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |  |  |
| Pre-set sub-array down-tilt (degrees) | 3 |  |  |
| Total TRP (dBm) | 37 for 100MHz For the calibration |  |  |
| Spatial isolation (dB) | TBD |  |  |
| Mechanical down-tilt (degrees) | 6 |  |  |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°. Note 2: The element gain includes the loss and is per polarization. Note 3: Void Note 5: For the case of 3 elements per sub array, dv will be 2.1 wavelengths. |  |  |  |

For TDM manner:

Antenna array parameters of IASC BS for 4GHz

| Parameter | Macro urban |  |  |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |  |  |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |  |  |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |  |  |
| Antenna polarization | Linear ±45º |  |  |
| Antenna sub-array configuration (Row × Column) | 4× 8 elements |  |  |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V |  |  |
| Number of element rows in sub-array | 3 |  |  |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |  |  |
| Pre-set sub-array down-tilt (degrees) | 3 |  |  |
| Total TRP (dBm) | Option 2: 52 for 100MHz for the calibration |  |  |
| Mechanical down-tilt (degrees) | 6 |  |  |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°. Note 2: The element gain includes the loss and is per polarization. Note 3: Void Note 5: For the case of 3 elements per sub array, dv will be 2.1 wavelengths. |  |  |  |

For full-duplex operation mode with different Tx/Rx panels:

Antenna array parameters of IASC BS for 7GHz

| Parameter | Macro urban |  |  |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |  |  |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |  |  |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |  |  |
| Antenna polarization | Linear ±45º |  |  |
| Antenna sub-array configuration (Row × Column) | Tx panel: 4 × 16 elements Rx panel 4 × 16 elements |  |  |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V |  |  |
| Number of element rows in sub-array | 3 |  |  |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |  |  |
| Pre-set sub-array down-tilt (degrees) | 3 |  |  |
| Total TRP (dBm) | 37 for 100MHz For the calibration |  |  |
| Spatial isolation (dB) | TBD |  |  |
| Mechanical down-tilt (degrees) | 6 |  |  |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°. Note 2: The element gain includes the loss and is per polarization. Note 3: Void Note 5: For the case of 3 elements per sub array, dv will be 2.1 wavelengths. |  |  |  |

For TDM manner:

Antenna array parameters of IASC BS for 7GHz

| Parameter | Macro urban |  |  |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |  |  |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |  |  |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |  |  |
| Antenna polarization | Linear ±45º |  |  |
| Antenna sub-array configuration (Row × Column) (Note 4) | 8× 16 elements |  |  |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V |  |  |
| Number of element rows in sub-array | 3 |  |  |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |  |  |
| Pre-set sub-array down-tilt (degrees) | 3 |  |  |
| Total TRP (dBm) | Option 2: 52 for 100MHz for the calibration |  |  |
| Mechanical down-tilt (degrees) | 6 |  |  |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°. Note 2: The element gain includes the loss and is per polarization. Note 3: Void Note 5: For the case of 3 elements per sub array, dv will be 2.1 wavelengths. |  |  |  |

For full-duplex operation mode with different Tx/Rx panels:

Antenna array parameters for 30GHz

| Parameter | Urban Macro |  |  |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 8.0 |  |  |
| Horizontal/vertical 3 dB beam width of single element (degree) | 65º for H 65º for V |  |  |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |  |  |
| Antenna polarization | Linear ±45º |  |  |
| Antenna array configuration (Row × Column) (Note 4) | Tx panel: 4 × 16 elements Rx panel 4 × 16 elements |  |  |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.5 of wavelength for V |  |  |
| Number of element rows in sub-array | 1 |  |  |
| Vertical element separation in sub-array (dv,sub) | 0.5 of wavelength of V |  |  |
| Total TRP (dBm) (Note 3) | 30dBm for 200MHz for the calibration |  |  |
| Spatial isolation | TBD |  |  |
| Mechanical down-tilt (degrees) | 3 |  |  |

For TDM manner:

Antenna array parameters for 30GHz

| Parameter | Urban Macro |  |  |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 8.0 |  |  |
| Horizontal/vertical 3 dB beam width of single element (degree) | 65º for H 65º for V |  |  |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |  |  |
| Antenna polarization | Linear ±45º |  |  |
| Antenna array configuration (Row × Column) (Note 4) | 8 × 16 elements |  |  |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.5 of wavelength for V |  |  |
| Number of element rows in sub-array | 1 |  |  |
| Vertical element separation in sub-array (dv,sub) | 0.5 of wavelength of V |  |  |
| Total TRP (dBm) (Note 3) | 30dBm for 200MHz for the calibration |  |  |
| Spatial isolation | TBD |  |  |
| Mechanical down-tilt (degrees) | 3 |  |  |

Transmission power control model for TN UE

Agreement:

For downlink scenario, no power control scheme is applied.

For FR2-1: from TR 38.803

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied with following parameters.

-	CLx-ile = 88 + 10*log10(200/X), where X is UL transmission BW (MHz)

-	γ = 1

For FR1:

Power control model is updated for the parameters.

where,

-	Pmax = 26dBm

-	CLx-ile = Pmax - Targeted UL SNR - BS Noise Floor

= Pmax - Targeted UL SNR - (-174 + NFBS + 10log10(BWUL) + 60)

Targeted UL SNR = 15dB for the calibration

NFBS = 6dB for 7GHz.

BWUL depends on the scheduled UE numbers.

-	γ = 1

-  Rmin=10^(( Pmin - Pmax)/10)

-  Pmin = -33dBm

Received power model

Agreement:

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

Other simulation parameters

Agreement:

Other simulation parameters for 4GHz and 7GHz

| Parameters | Urban macro |  |  |
| --- | --- | --- | --- |
| Channel bandwidth | 100MHz for 4GHz 100MHz, 200MHz and 400MHz for 7GHz |  |  |
| Scheduled channel bandwidth per UE (DL) | to align with 7GHz coexistence simulation assumption as captured in R4-260xxxx |  |  |
| Scheduled channel bandwidth per UE (UL) | to align with 7GHz coexistence simulation assumption as captured in R4-260xxxx |  |  |
| The number of active UE (DL) | Same as the number of BS beam |  |  |
| The number of active UE (UL) | Same as the number of BS beam |  |  |
| Traffic model | Full buffer |  |  |
| DL power control | NO |  |  |
| UL power control | YES |  |  |
| Handover margin | 3dB |  |  |

Other simulation parameters for 30GHz

| Parameters | Urban macro |  |  |
| --- | --- | --- | --- |
| Channel bandwidth | 200MHz for the calibration |  |  |
| Scheduled channel bandwidth per UE (DL) | 200MHz |  |  |
| Scheduled channel bandwidth per UE (UL) | 200MHz |  |  |
| The number of active UE (DL) | Same as the number of BS beam |  |  |
| The number of active UE (UL) | Same as the number of BS beam |  |  |
| Traffic model | Full buffer |  |  |
| DL power control | NO |  |  |
| UL power control | YES |  |  |
| Handover margin | 3dB |  |  |

Noise figure

Agreement:

Noise figure values for 6G sensing coexistence study

| Frequency | UE | BS |
| --- | --- | --- |
| 4GHz | 9dB | 5dB |
| 7GHz | 13dB | 6dB |
| 30GHz | 10dB | 10dB |

Co-existence simulation methodology for TN

Agreement:

For the calibration of communication performance of single network without adjacent channel interference, use the SINR calibration as starting point;

Co-existence simulation methodology for ISAC

Simulation metric

Agreement:

For the calibration of sensing performance of single network without adjacent channel interference, use the SINR calibration as starting point;

FFS for simulation metric:

  - For performance metric for 6G sensing, discuss the following metric and any down selection for the following metrics:

    - Detection Probability

    - False Alarm Probability

    - Horizontal Localization Accuracy

    - Vertical Localization Accuracy

    - Velocity Accuracy

Simulation methodology

Agreement:

Further discuss the following options next meeting:

Option 1: the close formulation for different sensing metrics

    - Formulation 1: Detection Probability

    - Formulation 2: False Alarm Probability

    - Formulation 3: Horizontal Localization Accuracy

    - Formulation 4: Vertical Localization Accuracy

Option 2: use the RAN1 simulation platform to generate the sensing results;